PH 317 Worksheet on Antenna Calculations.		MJM		1/26/04	Name_____________

The claim is that for various center-fed antennas of length d the current is given by

(1) 	I(z') = Io sin( kd/2 - k|z'|).

This current vanishes at each end of the dipole, z' = d/2.

The radiated power requires

(2)	-d/2+d/2 I(z') exp(-ikz' cos ) .

Use a change of variables to transform the integral

-d/2 0 dz' sin (kd/2 - k|z'|) exp(-ikz' cos )

to an integral which runs from 0 to d/2, and gets rid of |z'| .  Show all your work. Bore me with details.

When this is done, display the integral (2) over the whole antenna as an integral from 0 to +d/2. Put it in the form of trig functions.

Now make use of   sin a cos b = 1/2 [ sin(a+b) + sin(a-b) ] and elbow grease to show that the integral winds up in the form

d/2+d/2 I(z') exp(-ikz' cos )  = Iod [cos (u cos ) - cos(u)]/[u sin2()],

where u = kd/2.