Example 20

Figure 1.

Figure 2.

An non-inverting amplifier circuit is given in figure 1.

a) If the load resistorRL=1kΩ, determine vassuming the op amp is ideal. Repeat the analysis forRL=100kΩ.
b) Replace the operational amplifier by the finite gain model shown in figure 2. Assume the parameters of the op amp are R=100kΩ, R=100Ω, and A=100,000Repeat the analysis of a).

Firstly, for ideal op amp circuit, we are trying to find vo for two cases: RL=1kΩ and RL=100kΩ.

Secondly, the op amp is replaced by a more realistic model of op amp and we need to and determine the output voltage of the op amp.

Let’s work on problem a first. The ideal op doesn’t draw any current. So the two input currents are zero. The input voltages are equal vn=vp=10V  .

KCL equation gives us,

vo−vin10k+0−vom1k=0

vo10k−vin10k−vin1k=0

Move the two terms two the right side of the equation,

vo10k=vin10k+vin1k

Multiplied the equation by 10k at both sides,

vo=vin+10k1kvin=1+10k1kvin=11×10=110V

For problem b), ideal op amp is replaced by a finite gain model. The inverting terminal is connected with a 10V power supply through the 1k input resistor. The 10k resistor connects the inverting terminal with the output of the op amp.

Let us try to analyze the circuit by node voltage method.

vn1+vn−vo10+vn−vpRi=0  (1)

vo−A(vp−vn)Ro+vo−vn10+voRL=0 (2)

vp=10R=100, R=0.1, and A=100,000

vo=109.987VR=1kΩ
vo=109.988VR=100kΩ