Homework 2

ECE 207 – Electrical Engineering

Due: 9/13/2012

- A balanced three-phase wye-connected generator has an internal impedance of Zgen = 0.2 +

j0.5 Ω. The internal voltage of the generator is 120 V per phase. The generator feeds a

balanced three-phase wye-connected load having an impedance of Zload = 39 + j28 Ω. The

impedance of the line connecting the generator to load is Zline = 0.8 + j1.5 Ω. Calculate:

  - Van, Vbn, and Vcn at the terminals of the generator in phasor form (polar form)

  - Vab, Vbc, and Vca at the terminals of the generator in phasor form (polar form)

- A three-phase motor draws 20 kVA at 0.8 lagging power factor from a 220 V source. If the

motor is wye-connected, calculate:

  - Three-phase power P in kW

  - Single-phase power P in kW

  - Three-phase reactive power Q in kVAR

  - Single-phase reactive power Q in kVAR

  - Three-phase complex power S in kVA

  - Single-phase complex power S in kVA

  - Line and phase currents for each line and phase in Amps

- Repeat Problem 2, if the motor is delta-connected. Also, it is desired to improve the power

factor to 0.9 lagging by connecting a three-phase wye-connected capacitor bank across the

terminals of the motor. Determine the required three-phase kVAR rating of the capacitor

bank.

- A 345 kV transmission line delivers 500 MVA, 0.866 lagging power factor to a three-phase

wye-connected load. The line impedance that connects the wye-connected source to the load

is Zline = 0.5 + j1.5 Ω, and the load terminal voltage is 345 kV. Calculate:

  - Line and phase currents in Amps for one phase only

  - Source line and phase voltages in kV for one phase only

  - Voltage regulation and efficiency

- Repeat Problem 4, if the load is delta-connected.