PH 404 Helmholtz resonator introduction.			1/18/04

A helmholtz resonator acts like a mass-spring system, and it's pretty easy to work out.

For a mass-spring system,   = (k/m), so we will develop equations for m and k, and obtain an expression for .

A plug of air is taken to be oscillating in the neck of the resonator. (This actually must include a larger neck length L than the nominal value, to account for some air entering and leaving the neck area.). For a neck of length L and cross-sectional area A, the mass is

m =  L A.

Now, we need to recall the definition of bulk modulus, B

B = -p/(V/V) .

As the plug of air moves into the volume a distance x, the change in volume is V = A x.  The increased pressure inside, p, opposes this motion, and we have

p = - B (V/V) = - B x A /V

The force F opposing the motion is of course  F = A p, so we have

F = - x  B A2/V .

We can see the spring constant k shaping up as -F/x = A2 B/V .  Then we put the pieces together and have ( oops - we have to remember that c2 = B/ for  gas or liquid)

2 = k/m =  c2 A/(LV)

Or,

resonant = c (A/LV)

Dimensionally,  is 1/t, and c is length/t, so the square root must be 1/length. Check it out and you'll see that area/(length* volume) is 1/length^2, and things come out right.

(  is the angular frequency in radians/sec, and not the frequency f in Hz.)