ECE 370

HW SET #1

- A 60-Hz ideal transformer is rated 240/120 V. This transformer is connected to a 240 V source, and a load of ZL = 2 + j20 Ω is connected to the low voltage (output) side. Calculate:

  - Output (load) current

  - Output (load) real power

  - Output (load) reactive power

  - Output (load) apparent power

  - Output (load) complex power in rectangular and polar forms

- A 50-Hz ideal transformer is rated 240/120 V. This transformer is connected to a 240 V source, and a load of ZL = 2 + j20 Ω is connected to the low voltage (output) side. Calculate:

  - Load impedance referred to the high voltage (source or input) side

  - High side current

  - Real power supplied by the source

  - Reactive power supplied by the source

  - Apparent power supplied by the source

  - Complex power supplied by the source in rectangular and polar forms