ECE 370

HW SET #5

- Determine the speed of a 50 Hz synchronous generator, if it has:

  - 2 poles

  - 4 poles

  - 6 poles

  - 8 poles

- Determine the number of poles of a synchronous generator that is driven by a prime mover at 750 rpm to generate ac voltages at a frequency of:

  - 25 Hz

  - 50 Hz

  - 60 Hz. Is this possible?

- Determine the system frequency that is required to operate an 8-pole, 600-V synchronous motor at 750 rpm.

- Problem 4.1 of textbook.