Example 18

Design an op amp circuit such that vout=−3v1−5v2+4v3.

In this problem, we want to design a circuit having three inputs v1, v2, and v3 , and one output vout.The output must be related to the inputs by vout=−3v1−5v2+4v3. This required circuit must multiply each input by a number and add the results. The polarity of the first two input voltages are reversed.

As we know, one of the applications of operational amplifiers is to build circuits that perform mathematical operations. This is why it is called operational amplifier. So we can use op amps to design this circuit. A basic summing amplifier comes to my mind now. Let’s consider a 3 inputs summing amplifier circuit.

For a summing amplifier, the output should be a weighted sum of all the inputs. The weight for each input is simply the feedback resistance divided by the input resistance which is connected to the input.

vo=−RfR1vin1+RfR2vin2+RfR2vin2=−RfR1vin1−RfR2vin2−RfR2vin2

We might want to connect input v1, v2 to vin1 and vin2.  Notice here, the polarity of each input is reversed by the summing amplifier. In the design specification, we don’t want to flip the polarity of v3. So we might use an inverting amplifier to reverse polarity ofv3  first and then sent it to the input of the summing circuit.

Firstly, let’s connect the first two inputs of the summing amplifier with v1 and  v2. We can pick up a reasonable resistance value for the feedback resistor, which can be 15kΩ.

To make RfR1=3, R1should be Rf3=153=5kΩ

Similarly, the coefficient of v2 is negative 5.

To make RfR2=5, R2should be Rf5=155=3kΩ

Assume that we want the gain of the inverting amplifier to be negative 1,

to make RfR3=4, R3should be Rf4=154=3.75kΩ. This is not a commercial available component. We might have to use two resistors for this resistance. Let us make RfR3=1 and let the inverting circuit to provide a gain of negative 4.

RfR3=1 which gives us R3=15kΩ.

RoRin=−4. So The two resistance of the inverting amplifier can be set as Rin=1kΩ and Ro=4kΩ

Finally the output of the inverting amplifier is connected with the third input of the summing amplifier.

.