Example 7.

In the circuit, the switch closes at t=0. The capacitor is suddenly connected to a power supply, the voltage source can be modeled as a step function, and the response is called the step response.

Consider the RC circuit after the switch closes. The source voltage is a constant V. We want to determine the capacitor voltage.

We assume the initial voltage of the capacitor isV0.

Since the voltage of a capacitor cannot change instantaneously.

vc(0+)=vc(0−)=V0

Using the KVL law,

RCdvcdt+vc−V=0 dvcdt+1RCvc−VRC=0

Rearrange the terms:

dvcdt=V−vcRCor dvcdt=−vc−vRC

dvcvc−V=−1RCdt

vc(0)vc(t)dvcvc−V=0t−1RCdt

integrating on both sides:

ln(vc−V)|vc(t)vc(0)=−1RCt|t0  ln[vc(t)−V][vc(0)−V]=−1RCt

taking the exponential of both sides

[vc(t)−V][vc(0)−V]=e−1RCt

vc(t)=V+[vc(0)−V]e−1RCt=Vf+(V0−Vf)e−tτ, here τ=RC

Key steps to calculate the step response of a first order circuit:

- Find the initial voltage V0=vc(0+) at t=0+

and the final voltage Vf=vc(∞) as t→∞

- Find the time constant  τ=RC

- vc(t)=Vf+(V0−Vf)e−tτ