Example 9.

For the circuit shown below, the switch has been in position a for a long time and it is moved to b at t=0. Determine the initial and final values the inductor voltage and current iL(0+),vL(0+), iL(∞), vL(∞)

An instantaneously change in inductor current is impossible because the infinite voltage would be required. This means the inductor current must be continuous. vL=LdiLdt

iL(0+)=iL(0−)=109+1=1mA

vL(0−)=0V

vL(0+)=20−1×(1+1)=18V

For a circuit with dc source, if the circuit stays for a very long time without any switching taking place, we say that the circuit is in the steady state. In the steady sate mode, all the currents in the circuit are constants and their derivatives are zero. vL=LdiLdt=0. So the inductor behaves as a short circuit.

iL(∞)=201+1=10mA

vL(∞)=0V