Name: ___________________________   Section: ____

Practical 03

Read and perform the practical guide. Answer the questions as you work through the practical. Fit each answer within the templated text box area.

Sum (p2)

[4] (Need) What are the three uses of x5? Which lines of the program correspond to each of these three uses? Write a one-sentence description of what the instruction is doing.

[4] (Need) Where are main, loop, done, N and Sum located (i.e. what address)?

[4] (Performance) How many instructions does your modified program execute when N is equal to 5? How many additional instructions over the original is this? Can this be improved? If so, how?

[4] (Correctness) Will your modified program still work if N<0? You do not need to make any additional modifications, just answer conceptually.

Swap (p3)

[4] (Correctness) What is the value of max and maxindex at the end of the program? Are they what you expect?

[4] (Correctness) Comment out `slli x10, x7, 2` and re-assemble and run the program. What happens? Compare the address table between the program with and without `slli`. Which is correct and why?

[8] With the slli instruction still commented out, examine the value of x10 and its use in lw x10,0(x10). What does “Load address not aligned to word boundary 0x10010001” mean? Why did commenting out the slli instruction invalidate the program? Explain in the context of x10, x10's value, and how the memory segment is organized.

[4] (Need/Correctness) If you repeatedly apply your modified program to the subarrays of A from 0 to (N-i) where i is the number of times you've applied your program, what is the final state of A?

[8] (Iteration/Correctness) Like p2.asm this program doesn't work if N is equal to 0. It is brittle in other ways as well. For example, what happens if they are all less than -1? What if they are all -231, the max negative value? Is there a more robust way to ensure the max value is identified?

Sort (p4)

[4] (Need/Iteration) If SwapMaxWithLast needed to return two values --- the two values swapped in its execution --- for debugging purposes, what changes to p4-swap would need to be made to accomplish this?

[4] (Iteration) If SwapMaxWithLast needs to call another procedure in its execution, what changes to p4-swap would need to be made to accomplish this?

[4] (Iteration) If p4-loop needs to be converted into a procedure --- similar to what you did when refactoring p3 into p4-swap, what changes to p4-loop would need to be made to accomplish this?

Fib (p5)

[8] (Need) Look at the registers being used and notice that s-registers are being used to store both N and fib(N-1). An alternative is to use t-registers instead. What are the differences between using s and t-registers to implement fib? Are there any performance differences between the two? Which would you choose and explain why.

[4] With your chosen approach, how is your stack frame organized for each fib call? Complete the table illustrating a stack consisting of 3 frames, how stack frame is organized, the SP offset, and the content stored at each offset. Note that you will likely store more than 2 words of data in each frame, the table is only a starting point.

| Frame Number | SP offset | Data Content |
| --- | --- | --- |
| 1 | 20 |  |
|  | 16 |  |
| 2 | 12 |  |
|  | 8 |  |
| 3 | 4 |  |
|  | 0 |  |

[4] (Correctness) When you have correctly implemented fib, if you call fib(5), does it behave as expected? How many times is fib recursively called?

[8] (Correctness) In fib, you should have two internal calls to fib. Assemble and set a breakpoint at the line AFTER the first internal call to fib. Then, while calculating fib(5), observe the state of the stack when the breakpoint is hit (i.e., when recursion using the first jal ra,fib is complete).

Replace this textbox with your screenshot

Take a screenshot of the stack in RARS (Execute tab > Data Segment panel > dropdown Menu > current sp) and use colored boxes to box out each stack frame.

[4] (Correctness) What happens if fib does not restore the return address before using 
jalr x0,0(ra)?

[0] What is the git commit ID for your final commit of your code. This is required to pass the assignment. Check Practical 1 for instructions on how to get the correct commit ID.