Digilab 2E Local Guide

Ed Doering

January 28, 2003

- Documentation is located at http://www.digilentinc.com. Follow the “reference” tab, ignore the Java runtime error (if you get one), and look for the “Digilab 2E” entry. You can get a nine-page manual, a detailed schematic, and a simple reference design (a complete Xilinx ISE projection in a ZIP file).

- The FPGA is a member of the Spartan-IIE family (Spartan2E). The specific device is the XC2S200E-PQ208, with speed grade 6C. The “PQ” means “plastic quad flatpack” package, and the “208” means a total of 208 pins on the package.

- This FPGA has the following resources:

  - 200,000 max system gates

  - 5292 logic cells

  - 1176 CLBs (configurable logic blocks)

  - 56K block RAM bits.

  - User I/O pins is probably around 180 (I haven’t found the specific number yet).

- Specify the “xcs200e-6pq208” device to the Xilinx implementation tools.

- Download a bitstream as follows:

  - Ensure that the Digilab 2E board is powered (LED2 will be on)

  - Ensure that the Digilab 2E board is connected to the PC parallel port

  - Ensure that the switch SW1 is set to “JTAG” position (you may set it back to “PORT” position after downloading if your design needs the parallel port connection)

  - While in Xilinx ISE, double-click the “Configure Device (iMPACT)” subprocess contained in the “Generate Programming File” process.

  - Right-click the FPGA icon in the iMPACT main window, and select “Program…”

  - Click “OK”. Do not select the “Verify” option.

  - If you receive an error message about the JTAG clock, right-click the “Generate Programming File” process in the Xilinx ISE Project Navigator, select the “Startup Options” tab, and select “Startup Clock = JTAG Clock”. You should only have to do this once for a given project.

- After changing your source text:

  - Double-click the “Generate Programming File” process to create new bitstream file

  - Right-click the FPGA icon in the iMPACT main window, and select “Assign New Configuration File…”

  - Select the .BIT file associated with your project (NOTE: I experimented a bit and concluded that iMPACT only reads the .BIT file once. If you update the .BIT file in Xilinx ISE, then re-program the FPGA without doing “Assign New Configuration File”, you get the behavior of the old .BIT file)

  - Right-click the FPGA icon in the iMPACT main window, and proceed as before.