### Using Webpack ISE 5.1 (Service Pack 3) with Digilent FPGA Board

07 Feb 2003

This document outlines the essential differences from existing documentation for Xilinx Foundation Series 4.2i and XESS Corporation GXESSTOOLS for the XS-40.

Creating new project:

- Device Family = Spartan 2E

- Device = xc2s200e

- Package = pq208

- Speed Grade = -6

- Design Flow = XST Verilog

Working with UCF file:

- The UCF file must be added as its own source (do the same procedure as you would to add an existing Verilog file)

- Select your top-level module when asked for the source association

- To edit the UCF file within the Webpack ISE environment, expand the “User Constraints” process and double-click “Edit Constraints (Text)”

Dollar signs in Verilog names (do either of the following):

- Replace dollar signs in Verilog source with underscore (and ensure that the same names are used in the UCF file)

- Replace dollar signs in the UCF file only (the XST synthesis tool maps the ‘$’ character to ‘_’ character, so changing only the UCF file is sufficient)

To re-start implementation from scratch:

- Select “Project” followed by “Cleanup Project Files”

Creating the .bit file:

- Right click the “Generate Programming File” process, select “Properties”, select “Startup Options” tab, and select “JTAG Clock” for the “FPGA Start-Up Clock” (note that you only need to do this step one time for a given project)

- Double-click “Generate Programming File” to make the .bit file

Downloading the .bit file:

- Ensure that the Digilab 2E board is powered (LED2 will be on)

- Ensure that the Digilab 2E board is connected to the PC parallel port

- Ensure that the switch SW1 is set to “JTAG” position (you may set it back to “PORT” position after downloading if your design needs the parallel port connection)

- Expand the “Generate Programming File” process and double-click “Configure Device (iMPACT)”

- Click “Next” button three times

- Right-click the FPGA icon in the iMPACT main window, and select “Program…”

- Click “OK”. Do not select the “Verify” option.

Downloading an updated .bit file (assuming iMPACT is still running):

- Take care to avoid launching a second copy of iMPACT, since this will cause the parallel port connection to get loused up (you will probably need to reboot)

- Right-click the FPGA icon in the iMPACT main window, and select “Assign New Configuration File…”

- Select the .BIT file associated with your project (NOTE: I experimented a bit and concluded that iMPACT only reads the .BIT file once. If you update the .BIT file in Xilinx ISE, then re-program the FPGA without doing “Assign New Configuration File”, you get the behavior of the old .BIT file)

- Right-click the FPGA icon in the iMPACT main window, and proceed as before