ECE250 Equation Sheet Test 1  April 1, 2010 (KEH)

Diode Equation: Id=Is⋅(eVd(nVT)−1)  where Is = reverse saturation current. IsDOUBLES for every 10 degree C rise
                                                                                                                      Vd is the anode-to-cathode voltage and Id is current flowing into anode

Thermal Voltage:  VT = kT/q = 25.7 mV at room temperature T = 300K.

DC load voltage:  Vdc=Vm,s−0.7V−Vr2 (half-wave rect)   Vdc== Vm,s−1.4V−Vr2 (full-wave rect)

Half-wave rectifier ripple voltage:     Vr=(Vm,s−0.7V−Vr2)RLfsource⋅C                         Vm,s=peak source voltage

Full-wave diode bridge rectifier ripple voltage:      Vr=(Vm,s−1.4V−Vr2)RL2fsource⋅C   Vm,s=peak source voltage

Full-wave diode bridge peak diode current:  Idmax=VmRL⋅(1+2π)⋅Vm2⋅Vr, where Vm=Vm,s−2(0.7V)

Small-Signal ac Model of Diode:  rd=n⋅VTIdQ, where IdQ is the dc (quiescent) component of the diode current.

Plotting Load Lines over nonlinear element’s IV curve:  First find Thevenin equivalent “looking out” from
 the terminals of the nonlinear element. Then plot load line: IINTERCEPT = Vth/Rth, and VINTERCEPT = Vth

Multiple Diode Analysis using 0.7 V battery model of diode:  Define Id’s flowing into each anode.  Define Vd’s anode-to-cathode.  Guess which diodes are ON and replace them with 0.7 battery, replace OFF diodes with open circuit.  Analyze circuit.  Check to ensure diodes that are assumed ON have Id > 0, and diodes that are assumed OFF have Vd < 0.7 V.

Carrier Concentration in Intrinsic Si (1/cm3):  ni2=BT3e−EgkT

at T=300K:  B=5.4x1031/(K3cm6), Eg=1.12 eV, k= Boltzmann’s Constant = 8.62x10-5 eV/K, ni=1.5 x 1010 1/cm3
Diffusion Current Density (A/cm2):             Jp=−qDpdpdx      Jn=qDndndx       q = 1.6 x 10-19 Coulombs
							  	                           Dp = 12 cm2/s, Dn = 34 cm2/s

Drift current Density (A/cm2):                         Jdrift = q(pμp+nμn)E

Resistivity (Ω-cm) and Resistance (Ω):          ρ=1[q(pμp+nμn)]]                    R = ρL/A

Carrier Concentration in n-type Si (1/cm3):  nn0=NDpn0=ni2ND  in p-type Si (1/cm3):  pp0=NAnp0=ni2NA

Built-In Junction Voltage (V):                          Vo=VTln(NANDni2)

Depletion Region Capacitance (F):  	Cj=Cj0(1+VRVo)m   where Cj0=εSiA(Wdepletionregion)VR=0

and  m = junction grading coefficient = 1/3 to ½,

also note VR is diode’s CATHODE to ANODE voltage = -Vd