ECE250 HW 1 (Pages refer to Sedra and Smith (SS) 6th Edn)
Due in class on Tuesday, September 7, 2010

General instructions for preparing all ECE 250 homework assignments:

- Please use standard ECE homework format on all ECE250  homework assignments.

- Be sure to mark the homework number (in this case HW #1) prominently on the header page, so that you can keep your ECE250 homework submissions in order as they are returned to you.

- Because many of these review problem solutions are quite short, you may put more than one problem on a sheet of paper. But in later assignments when the problems are longer, please put one problem on each sheet of paper.

- Since the answers to each of the problems are given, you must show the relevant calculations that lead up to that answer.  No credit can be given for just writing down the answer or just writing down the problem statement!

- Remember that a picture is worth a thousand words --- therefore, wherever relevant, please show neat schematic circuit diagram(s) with all voltages and currents that are used in your accompanying equations marked on your schematic diagram to help document your solution.  You must do this in any problem that refers to a circuit diagram!

Review Questions (End of Chapter 1)

- SS1.2 (b) & (f)  (Ans: 1.6 W =>2 W rating, 0.121 W => 1/8 W, though ¼ W might be safer, considering the possibility of resistor value error tolerances.)

- SS1.3 (b) (d) (e) (Ans: 10 mW & 10 kΩ; 10 V & 1 kΩ; 31.6 mA & 31.6 V)

- SS1.11 (Ans: Put R = 250 Ω across the load resistor)

- SS1.17 (Ans: defining the current through R5 as flowing from (2) to (1), I5 = 1.91 μA and V21 = 5.7 mV)

- SS1.18 do first part only, with all 5 resistors = 1 kΩ (Ans: 1 kΩ)

- SS1.21 (b) & (d) (Ans: (717.0 – j450.5) Ω, (100 + j628.3) Ω

- SS1.23 (Ans: 28.6 kΩ, 38.6 mV, 1.35 μA)

- SS1.24 (Ans: 10 kΩ)

- SS1.27(f) (Ans: 6.28E6 r/s, 1 MHz)

- SS1.28(d) (Ans: 311 kV)

- SS1.31 Hint: First review the Fourier Series expansion of a square wave presented in SS-Section 1.2.  (Ans: 14 kHz, 441 mV, 312 mV rms, 693 mV pp, 71.4 μs)

- SS1.42 (a) & (b) Hint: You may want to first read about saturation in SS-Section 1.4.  (Ans: (a) vin(t) sine wave has 9 mV peak, (b) vin(t) sine wave has 57.5 mV peak)  Hint: for Part (b): consider just the first quarter of the output sine wave cycle of unknown amplitude Vp.  Note that for the output sine wave to be clipped 90% of the time, the output voltage must reach the clipping level (= 9 V), when the phase of the sine wave is 10% of 90 degrees = 9 degrees.  You should be able to set up an equation and solve it for the necessary value of Vp.

Chapter 3 Semiconductor Physics

Use the values given in Table 3.1 of SS Text (p. 159-160)

- P. 129 SSEx3.1(Answers given)

- P. 132  SSEx3.2 (Answers given)

- P. 133 SSEx3.3 (Answers given)

- P. 135 SSEx3.4 (Answers given)

- P. 137 SSEx3.5  (Answers given)

- P. 160 SS3.2 (ni = 2.2E6 carriers/cm3.

- P. 160 SS3.3 (pp = 1E18cm-3, np = 225 cm-3)

- P. 160 SS3.4 (pn = 1.5E3 cm-3, nn = 1.5E17 P atoms/ cm3)