Example (4)  Reverse-active NPN BJT, that was not covered on Quiz

Assume BJT has betaF = 100, betaR = 5, VECsat = 0.1 V, and VBCon = 0.7 V.

Note that the circuit below looks just like example (a) that was worked in class… except that NOW the emitter and collector have been interchanged!  Because Vin is > 0.7 V,  it looks quite likely that the BC junction is fwd biased, and the BE junction may be either reverse biased (=> reverse active) or forward biased (=> saturated).  We always begin by assuming the active case first, then seeing if this leads to a contradiction.   Except now we check the VEC voltage instead of VCE, and now we can assume that Ie = betaR*Ib instead of Ic = betaF*Ib…. because the role of the C and E terminals has been interchanged!

We shall assume reverse active mode.  Then Ie = betaR*Ib.  But Ib = (5-0.7)/10kohm = 0.43 mA.

This would make (assuming reverse active mode) the output voltage

Vo = Vcc – Re*(betaR)*Ib = 2.85 V.

Because this result came out > VECsat = 0.1 V, we can assume the BJT was INDEED reverse active, and we may claim that indeed, Vo = 2.85 V.

However, if Vo came out less than 0.1 V, we would have had to say Vo = 0.1 V (VECsat) instead.

---- K.E. Hoover