EC250 Devices and Modeling

Laboratory #3.  Diode Circuits and PSPICE

December 16, 2009 (KEH)

# Dept. of Electrical and Computer Engineering

# Rose-Hulman Institute of Technology

## Lab Team Members: ________________________    _______________________

Date Performed:  _________________________    Lab Station: ______________

Your lab report will consist of this document with all requested blanks filled in and all of the requested exhibit document, each must be labeled with exhibit number and descriptive caption, and stapled in the proper order to the back of this document.

Parts Needed: 120 V rms to 12 V rms step-down transformer (not bought, but temporarily checked out from instrument room), Four 1N4004 power diodes, Two 47-μF, 50 V electrolytic capacitors, One 2.2-kΩ and one 10- kΩ, ¼ W resistor, one breadboard, wire strippers.  Also a laptop PC is needed that has the Agilent USB image capture software.  

This scope capture software should already be set up on your laptop from your previous lab courses, but it can be re-installed by following the directions at \\tibia\public\Course Software\Agilent\Intuilink.  If your laptop PC does not have this Agilent USB Scope Capture software, please bring your laptop to the lab (B105) and install this software before our scheduled lab period, or bring a floppy disk to store your scope images on.  A “floating” USB floppy disk drive should be available in the lab to aid in transfer of the images to your laptop.  There will not be time to install the Agilent scope capture software during the lab time.

Caution: Do not to allow the 12 V rms output terminals of the step-down transformer to touch after it is plugged in, as this will burn out a fuse inside the transformer that is not repairable.  Please be careful to connect transformer to the power line ONLY AFTER it has been wired into the circuit, and the circuit has been carefully checked by your lab partner.  Please use only one scope probe (connected to Channel 1) in this lab, to avoid “ground faults” in the full-wave rectifier circuit.

1. Half-Wave Diode Rectifier

A.  Connect the 12 V rms secondary winding of a 120 V rms, 60 Hz step-down transformer in series with a 1N4004 power diode and a 2.2 k resistor, as shown below.   Remember that that the peak value (amplitude) of a sine wave is equal to the rms value of the waveform times the square root of 2.  Use an oscilloscope to observe first the transformer secondary voltage vs(t), and then later the voltage across the 2.2 k load resistor, vo(t).  In Exhibit A present an oscilloscope “screen shot” of vs(t) and also a separate screen shot of vo(t).   Each oscilloscope plot should show about two cycles of variation.  Your screen shots must display the “max” amplitude excursion, the “min” amplitude excursion, and the waveform frequency (which should be close to 60 Hz), using the scope’s “Quick Measure” feature.  Be sure that your oscilloscope input is configured for the X10 Probe Sensitivity mode, and that it is “dc-coupled”, so that the dc offset can be observed in the vo(t) waveform.  (This precaution will be equally important in the other parts of the laboratory as well.)  Recall that this is done by depressing the lighted Channel 1 button, so that the “Channel 1 Menu” appears.  Then press the left-most softkey until “Coupling DC” appears if it is not already displayed.  Also, press the right-most softkey labeled “Probe” to bring up the “Channel 1 Probe Menu”.  If “Probe 10:1” is not currently displayed, turn the selector knob that is located just to the right of the display screen, until it is displayed.

Record the peak value of the source voltage vs(t) and the peak value of the half-wave rectified output waveform, vo(t) below.  Note that Vs,max may be several volts different from the “expected peak value” if the transformer were delivering exactly 12 V rms.  This is because the so-called “120 V rms, 60 Hz” line voltage generated by the power company typically varies between 110 - 130 V, rms.  Also, it is normal to observe a slight dc offset at the output terminals of the transformer, making |Vs,max| a few tenths of a volt different from |Vs,min|.  You need not worry about this small non-ideality.

         Vs,max = __________     (compare this with 12*sqrt(2) Volts = 16.97 Volts  %diff = _____

Vo,max = __________    (Should be about 0.7 V less than Vs,max)

B. Perform a PSPICE transient analysis of this circuit, including at least two cycles of output variation.  When specifying the amplitude of your VSIN ac source, use the observed peak voltage, rather than the expected peak voltage 12*sqrt(2) value. Include your PSPICE schematic, and also a plot of the simulated vs(t) and vo(t) waveforms (plotted on the same set of axes), as Exhibit B. Be sure to use the cursor “mark label” button to mark the maximum and minimum points on both the vs(t) and vo(t) simulated “Probe Plot” waveforms, as was discussed in the “Getting Started with ORCAD PSPICE” handout. NOTE: As with all exhibits you include in ECE250 lab reports, please be sure to clearly label this exhibit along with a descriptive caption such as “Exhibit B.  PSPICE Schematic and Transient Analysis Plots of Half-Wave Rectifier Circuit.”  Comment on the similarity and any differences noted between the observed and simulated waveforms in the space below.

Subjective Comparison of Measured and Simulated Waveforms:

- Now connect a 47-F electrolytic capacitor across the 2.2 k load resistor.  Observe the polarity of the electrolytic capacitor!  Small-valued capacitors are often non-polarized (their terminals may be connected either way), but an electrolytic capacitor manages to achieve a large value of capacitance in a small package through the use of an electro-chemically formed dielectric, which requires a dc voltage, or “dc bias”, to be applied in a certain direction across the capacitor in order to keep the dielectric insulating layer formed.  In many common electronic circuits, such as this one, there is no problem in meeting this requirement, since we have an AC waveform “riding on top of” a dc (average) voltage level.  Therefore, the electrolytic capacitor’s (+) terminal should be connected to the node with the higher average (dc) potential.  When electrolytic capacitors are drawn in a schematic, the designer should be sure to include the required terminal polarity markings on the capacitor symbol.  If an electrolytic capacitor is accidentally connected backward, an unwanted chemical reaction ensues, and the capacitor can literally BLOW UP in your face!

Include an oscilloscope screen capture of the resulting filtered output waveform vo(t) as observed on the (dc-coupled) oscilloscope as Exhibit C for two input cycles  As before, your screen shot must display the “max” amplitude excursion, the “min” amplitude excursion, and the waveform frequency (which should be close to 60 Hz), using the scope’s “quick measure” feature.    From this vo(t) waveform, determine the ripple voltage Vr = Vomax - Vomin, which is the peak-to-peak variation in vo(t), and also calculate the observed percent output voltage ripple, which is (Vr /Vdc)*100%, where Vdc is the average of the peak and minimum vo(t) value = (Vomax + Vomin)/2.

Observed Vr = ___________        % Voltage Ripple = ________  Ripple Freq = _____ Hz

- Now “hand calculate” the ripple voltage, Vr, as derived in the class notes for the half-wave rectifier (see your class notes).  Assume that Vm = Vo,max recorded in Part 1A (which should be about one junction drop below Vs,max).  Then hand calculate the percent ripple, which is the ratio of the predicted worst-case value of Vr to Vdc = Vm – Vr/2 = (Vomax + Vomin)/2, expressed as a percentage.   Show your calculations in the space below.

Half-Wave Rectifier Voltage Ripple and Percent Ripple Prediction Using Hand Calculations

- Perform a PSPICE “transient” simulation of the circuit of Part C, where the simulation is run over two 60-Hz cycles.  Do not simulate the transformer.  Simply use a VSIN source that has its Voffset = 0 V, frequency = 60 Hz, and its (peak) amplitude set to the value of Vs,max that was measured in Part 1A.  Obtain printouts of the PSPICE simulation schematic, and the resulting PROBE plot of vo(t).  Attach these as Exhibit D to the end of this document.  Read the peak and minimum values of vo(t) using the PROBE cursor button, as was emphasized in the PSPICE familiarization lab, and select the correct waveform.  Use the “mark label” button to display the maximum and minimum values of vo(t) in your PROBE plot.  Once again, calculate the percentage ripple from your PSPICE PROBE plot in the space below. Compare the three different percentage ripple figures (observed, hand calculated, and PSPICE analysis).  Is the hand calculation or the PSPICE result closest to the observed percent ripple?

Summary for the Half-Wave Rectifier with Capacitor Filter
(1) Measured Vr	      ________ V    	Measured % Ripple:            _______%

(2) Hand Calculated Vr = ________ V          Hand Calculated % Ripple: _______%
(3) PSPICE simulated Vr = _______V          PSPICE simulated % Ripple ______%

2. Full-wave Diode Bridge Rectifier

Repeat steps A-E of Part 1 for the full-wave bridge rectifier circuit shown below.

CAUTION: Be careful to connect the diode bridge properly.   Before applying the power, check it carefully.  Then have your lab partner check it carefully, since if any one of the diodes in the bridge circuit is accidentally reversed, you will “short out” the power transformer (which is internally fused, but is not repairable)  It is normal for the 2200 ohm load resistor to run warm, since it is a ¼ W resistor that is required to dissipate approximately 202/2200 = 0.18 W (as heat).  

Also, please note that both the oscilloscope probe and the oscilloscope ground clip must be moved when going between the vs(t) and the vo(t) measurements for the full-wave bridge circuits.

Please do not try to display both vs(t) and vo(t) at the same time on the oscilloscope for this full-wave bridge rectifier, since the ground clips of the two different scope probes  would have to be connected to two different circuit nodes, effectively causing a short circuit, with accompanying sparks and smoke!

A. Build and test full-wave bridge circuit shown above.  Results for full-wave bridge without capacitor input filter must be filled in below:

Vs,max = __________     (compare this with 12*sqrt(2) Volts = 16.97 Volts  %diff = _____

Vo,max = __________    (Should be about two junction drops (1.4 V) less than Vs,max)

Attach Exhibit E  (corresponding to Exhibit A).

B.  Perform PSPICE transient analysis of full-wave bridge circuit shown above.  Set the VSIN source amplitude to the value of Vs,max observed in Part 2A. Attach Exhibit F (corresponding to Exhibit B).

Subjective Comparison of Measured and Simulated Waveforms:

C. Build and test full-wave rectifier with capacitor input filter added, as shown below.

Observed Vr = ___________        % Voltage Ripple = ________  Ripple Freq = _____ Hz

Attach Exhibit G (corresponding to Exhibit C).

D.  Full-Wave Rectifier Voltage Ripple Prediction and % Ripple using Hand Calculations (use the equations developed in the class notes.  Note that the ripple voltage Vr for the full-wave rectifier should be approximately ½ that for the half-wave rectifier, assuming the same source, load resistance, and filter capacitor.  Use the value of Vo,max that was observed in Part 2A for Vdc.

Full-Wave Rectifier Voltage Ripple and Percent Ripple Prediction Using Hand Calculations

E.   PSPICE Simulation of full-wave rectifier with capacitor input filter.
      Attach Exhibit H (corresponding to Exhibit D).  Show your PSPICE simulation percentage

ripple calculations in the space below.

Summary for the Full-Wave Rectifier with Capacitor Filter

(1) Measured Vr	      ________ V    	Measured % Ripple:            _______%

(2) Hand Calculated Vr = ________ V          Hand Calculated % Ripple: _______%
(3) PSPICE simulated Vr = _______V          PSPICE simulated % Ripple ______%

3. Full-wave Voltage Doubler Circuit

A full-wave voltage doubler circuit can be built by replacing two of the diodes in the full-wave diode bridge rectifier with capacitors.  The operation of circuit will be explained at the beginning of the lab period.  Use two 1N4004 power diodes, the 12 V rms transformer, two 47-uF electrolytic capacitors, and a 10 k, ¼ W load resistor.  Be careful to observe the electrolytic capacitor terminal polarities.

Note the change in the load resistor.  This is because the 2.2 kΩ, ¼ Watt resistor that we used as a load in the previous parts of this lab would get too hot in this application, since the output voltage will now be approximately doubled.  The power dissipated (as heat) in the 10 k load resistor is approximately 402/10000 = 0.16 W.  Capture the observed vo(t) waveform as Exhibit I.  Your screen shots must display the “max” amplitude excursion, the “min” amplitude excursion, and the waveform frequency (which should be close to 120 Hz), using the scope’s “Quick Measure” feature.  (Note that the output is approximately double that expected from the full-wave rectifier circuit studied earlier.)  Use Quick Measure to display the frequency and the peak-peak (ripple) voltage variation in Vo(t) in your screen shot.

 			          Voltage Doubler Circuit Diagram

From the observed vo(t) screen shot obtained above, you may calculate Vdc = (Vmax + Vmin}/2, the ripple voltage Vr = Vmax – Vmin, and the “% voltage ripple” = (Vr / Vdc)*100% = (Vmax – Vmin) / {{Vmax + Vmin}/2}*100% in the blanks below:

                                                Observed (Measured) Results
      Vdc = _______ V            Vr = _________V       % Voltage Ripple = _________

Next run a PSPICE transient simulation on this circuit, including your schematic and vo(t) PROBE plot as Exhibit J.  From your simulated plot of vo(t), use the cursor option to measure Vdc and Vr, and enter the results below.   Use the “Mark Cursor” option to mark and display the values of the minimum and maximum voltage excursions on your plot.

PSPICE Simulated Results

Vdc = _______ V            Vr = _________V       % Voltage Ripple = _________