Floppy Disk Setup Instructions

For Scopes in D115 (Agilent 54622, Mixed Signal Scopes)

- Press Utility button (hard button on panel)

- Hit “Print Config”  --- Soft Key

- Print to: Disk  --- Leftmost (Number 1) Soft Key – use it to select “Disk” option

- Format: TIF image --- Number 2 Soft Key – Use it to select “TIF” image option.

- Insert Floppy, make sure QUICK MEASURE is displaying desired values on screen

- Hit Quick Print button to save image.