ECE250 Lab 4 Addendum

- AM Modulation setup instructions for the Agilent 33120 15 MHz Function Generator
(This generator is used in room B105)

We desire to set up 50% AM modulated wave (with a 5 kHz carrier frequency and a 200 Hz modulating frequency) using the Agilent 33120, 15 MHz function generator.

    - Connect the scope to the function generator, so you can see the results of your setup.

    - Turn on the power to this unit.

    - Carrier Wave Setup

      - Press the “~” (sine wave) button to set create the sinusoidal carrier wave.

      - Set Frequency = 10 kHz (Press “Freq” key, “Enter Number” key, 10, kHz key)

      - Set Amplitude = 10 Vpp (Press “Ampl” key, “Enter Number” key, 10, Vpp key)

      - Set DC Offset = 0 Vdc (Press “offset” key, “Enter Number” key, 0, Vpp

    - AM Modulation Setup

      - Start AM modulation. (Press the blue shift key, then the “AM” key)

      - AM frequency = 200 Hz. (Press the blue shift key, then the “Freq” key, then the “Enter Number” key, 200, then the Hz key.

      - AM Depth = 50%  (Press the blue shift key, then the “Ampl” (level) key, 50, then press the “Enter” key.

    - Turn OFF AM Modulation before going on to the next part of the lab.

      - Press the blue shift key, then the “AM” key to toggle off AM modulation.