ECE331 HW #1  (Due in class on Monday, September 13)

Name: __________________________   Mailbox:________

1.  The following program is executed in sequence.  Fill in HNZVC flags, and also the accumulator register “A” contents, after each arithmetic instruction executes shown below.  If “H” is not affected, be sure to keep it at the value it was the last time it WAS affected!  (Please attach this sheet to your homework, with the blanks filled in, BUT also show your HEXADECIMAL math for each problem off to the right, as was done in class.

Start:	ldaa  #$15

adda #$38	HNZVC = ____________   A = $_______

ldaa  #$6A

adda #$59 	HNZVC = ____________   A = $_______

ldaa  #$FC

adda #$04 	HNZVC = ____________   A = $_______

ldaa  #$25

adda #$B7 	HNZVC = ____________   A = $_______

ldaa  #$6E

adda #$AD 	HNZVC = ____________   A = $_______

ldaa  #$81

adda #$FC 	HNZVC = ____________   A = $_______

ldaa  #$72

suba #$6E 	HNZVC = ____________   A = $_______

cmpa #$60 	HNZVC = ____________   A = $_______  (Note this is cmpa!)

cmpa #$70 	HNZVC = ____________   A = $_______  (Note this is cmpa!)

suba  #$A0 	HNZVC = ____________   A = $_______

2. (A) Study the assembly program below to make sure you understand how it adds two 32-bit numbers and then calculates “5 factorial” = 5*4*3*2*1 = 120.  Then hand assemble this program using your CPU12 pdf manual to find the OP CODES.  Fill in the Addr and Contents columns on the left, just as an assembler would do.  I have started you out by filling in the first few Addr and Contents entries.  Please do NOT cheat and use the CodeWarrior assembler at this time!

Addr    Contents	;*** Homework #1 Problem 2 (Hand Assembly)

; export symbols

XDEF Entry        ; export 'Entry' symbol

ABSENTRY Entry    ; for absolute assembly: mark this as application entry point

RAMStart    EQU  $0400  ; absolute address of start of on-chip RAM ($0400 - $0FFF)

ROMStart    EQU  $4000  ; absolute address of on-chip Flash ROM

ORG RAMStart

0400                                    ResultWd:   RMB 4        ; RMB 4 is an assembly directive =>reserve 4 memory bytes.

0404                                    TEMP:       DS.B 1          ; “RMB” is the same as "DS.B"

0405                                    FACT_RSLT:  DS.B 1

ORG ROMStart

;Adding two long words

4000  B6 40 3F       Entry:	LDAA LongWd1+3	 ; Add least significant bits

4003  BB 40 43                  ADDA LongWd2+3

4006  7A  04 03                 STAA ResultWd+3	 ; Store result

LDAA LongWd1+2

ADCA LongWd2+2	 ; Add next byte with CARRY from last addition stage

STAA ResultWd+2

LDAA LongWd1+1

ADCA LongWd2+1	 ; Add next byte with CARRY from last addition stage

STAA ResultWd+1

LDAA LongWd1

ADCA LongWd2		 ; Add next byte with CARRY from last addition stage

STAA ResultWd

;Calculating 5 factorial 5*4*3*2*1 = 120 (not very efficient pgm!)

MOVB #5,TEMP    ;Put the value to calculate factorial of into TEMP

LDAB TEMP

NXT_MULT:   	DEC  TEMP

BEQ  DONE	 ;When TEMP decremented down to 0, we are done!

LDAA TEMP

MUL		    ;8-bit unsigned multiply with 16-bit result (A:B = A X B)

BRA  NXT_MULT

DONE:    	STAB FACT_RSLT

HANG_HERE:    BRA HANG_HERE  ; End program with this endless loop

LongWd1:    	FCB $12, $34, $56,$78                  ;FCB => form constant byte.

LongWd2:    	DC.B  $0A, $BC, $CD, $DE          ;FCB Equivalent to “DC.B”

ORG $FFFE

FDB     Entry      ; Init Reset Vector.  FDB => Form Double Byte

Equivalent to “DC.W”

(B) Determine the contents of each of the RAM locations $0400 through $0405 after the program execution has reached the “dynamic halt” loop at location “HANG_HERE”.

(C) Now use the CodeWarrior IDE to assemble.  Verify your assembly results of Part A by clicking on Project – Disassembly.  Bring up the Full Chip Simulator.  Be sure to single step through the program carefully, and verify that the results claimed in Part B are indeed displayed in the memory window of the simulator after the program has completed  Attach to this HW document a screen shot of your simulator results (ALT – Print Screen) that clearly shows the correct results in the memory window.

Also print out the .LST file that contains the results of the machine assembly process.  This can be done CodeWarrior by clicking on the Project tab, and then selecting Disassemble.  Be sure to check each machine code that was generated by the assembler in this .LST file to make sure that you have PERFECT agreement with your hand assembled results from Part A.