;*********************************************************

; ECE331 HW 2 Addressing Mode Homework

;  NAME: _____________________________	 Box #___________

; This is an 9S12C32 Assembly Program

; Fill in ALL blanks in the comment fields

; with the proper value(s) AFTER the

; commented line has executed.

;**********************************************************

XDEF Entry        	; export 'Entry' symbol

ABSENTRY Entry    	; for absolute assembly

ORG $0800		;$0800 = Start of RAM

var1:      	rmb 1              	;(Reserve memory byte = DS.B)

vararray1: 	rmb 10

var2:      	rmb 1

ORG $4000			;$4000 = Start of Flash ROM

const1:    fcb 3,2,7,8      		;(Form constant byte = DC.B)

const2:    fcb %10011010,%01011100  	;"%" => binary

const3:    fcb 'ECE331',0	  	;(Note ASCII strings can be allocated)

const4:    fcb $0b,$ad,$fa,$d0

const5:    fdb $0bad,$deed  		;(Form double byte = DC.W)

adrconst1: fdb const1

adrconst2: fdb const2

adrconst3: fdb const3

PTT:     EQU $240			;Port T Data I/O Register

DDRT:    EQU $242			;Port T Data Direction Register

Entry: ldd #4000    	 ;d = $_______

;b = $____ a = $____ (careful!)

ldd #$4000    	;d = $_______

ldd $4000     	;d = $_______

ldab #-1	    	;b = $______

ldab #$ff	  	;b = $_____

ldy Entry	  	;y = $______

ldy #Entry      	;y = $______

ldy #Entry-(3*2)	;y = $______

ldx const1      	;x = $_____

ldaa const1+2   	;a = $_______

ldaa  #$55  		;Note that assembler is case insensitive

staa  DDRT  		;DDRT contains $_____

ldaa  #%01101100 	;Note that the "%" prefix => binary

staa PTT 		;List the four Port T pins

;that are output pins, and indicate

;their value

;___________  ____________  __________   __________

;Assume outside world drives the

;the following input signals into

;the four PTT input pins:

;PT7 = High, PT5 = Low, PT3 = High, PT1 = Low.

ldaa PTT 		; a = $______

ldx #const2

ldaa 0,x  		;x = $______, a = $_______

ldd  0,x  		;x = $______, d = $_______

inx

ldab 0,x  		;x = $______, b = $_______

ldy #const4

ldaa 1,y  		;y = $______ a = $_____

ldd  2,y   		;y = $______ d = $________

ldy  3,y   		;y = $______

ldy  #2

ldy  const1,y   	;y = $_______

ldy  0,pc       	;y = $_______	;Recall PC points to op

ldy  3,pc      	;y = $_______	;code of next instruction.

ldx #const4

ldaa 1,+x       	;x = $_______  a = $______

ldaa 1,+x       	;x = $_______  a = $______

ldx #const4

ldaa 1,x+      	 ;x = $_______  a = $______

ldaa 1,x+       	;x = $_______  a = $______

ldx #const5

ldd  2,-x       	;x = $_______  d = $______

ldd  2,-x       	;x = $_______  d = $______

ldx #const5

ldd  2,x-       	;x = $_______  d = $______

ldd  2,x-       	;x = $_______  d = $______

ldy  #var2

ldd  #$1234

std  2,-y       	;y = $_______

ldd  const2     	;d = $_______

std  2,-y       	;y = $_______

ldd  const3     	;d = $_______

std  2,-y       	;y = $_______			<-------Fill in the contents

ldd  2,y+       	;d = $_______, 	y = $______	of RAM (below) after

ldd  2,y+       	;d = $_______, 	y = $______	this instruction executes

ADDR   Contents

ldd  2,y+       	;d = $_______, 	y = $______	$080B   -------

ldx  #const3						$080A	_____

ldaa #2							$0809	_____

ldab  a,x       	;a = $______, b = $______		$0808	_____

inca							$0807   _____

ldab  a,x      	 ;a = $______, b = $______		$0806   _____

ldd  #2				                        	$0805   _____

ldab  d,x       	;d = $______, b = $______

ldx  #0

ldaa [adrconst1,x] ;x = $______, a = $_____

inx

inx

ldaa [adrconst1,x] ;x = $______, a = $_____

inx

inx

ldd [adrconst1,x]  ;x = $_____, 	d = $______

ldd  #adrconst1

ldx #0

ldy [d,x] 		;x = $_______, y = $_______

inx

inx

ldy [d,x]  		;x = $_______, y = $_______

ldaa #1

ldab #2

ldx  #3

aba        	 	;b = $_____  	a = $______

nop	  	;b = $_____  	a = $______

abx        	 	;b = $_____  	x = $_________

exg a,b     		;a = $_____  	b = $______

inx         		;x = $_________

dex         		;x = $_________

ldaa #3

ldab #0

ldx #const4

here: addb a,x

deca

nop

nop

nop

bne here  		;Disp byte after OPCODE = $____

bra skip  		;Disp byte after OPCODE = $____

nop

nop

nop

nop

skip:	nop	 	;a = $____, b = $_____

done:	bra done 	;Disp byte after OPCODE = $____

;**************************************************************

;*                 Interrupt Vectors                          *

;**************************************************************

ORG $FFFE

fdb     Entry      ; Reset Vector initialized with starting address of program