# Handout for EC331 Lab #3:

# -- ASCII-Encoded Keyswitch Entry Terminal

Jianjian Song and Keith Hoover

Department of Electrical and Computer Engineering

Rose-Hulman Institute of Technology

Revised August, 2007

1	Objectives	1

2	Deliverable	2

3	Reference Materials and Equipment	2

3.1	References	2

3.2	Equipment	2

4	Introduction	2

5	Modular Programming	2

5.1	Software modules	2

5.2	Top-down modular design	3

5.3	Documenting a software module: the header block	3

6	Step 1: Write a variable-time-delay subroutine “wait_y_667ns”	4

6.1	Software interface specification for subroutine “wait_y_667ns”	4

6.2	Using the output compare feature of the 9S12C32’s hardware timing module	4

6.3	Verification of subroutine “wait_y_667ns”	6

6.4	Lab Report Part 1	8

7	Investigating the need for software switch debouncing: LED toggling program	8

7.1	Structure of LED toggling program	8

7.2	Running the LED toggling program with and without contact debounce delay	9

7.3	Lab Report Part 2	9

8	Debounced keyswitch array input subroutine “getkey”	9

8.1	Waiting for Port T changes to settle	14

8.2	Lab Report Part 3	14

9	Write an asynchronous serial output routine “outchar”	14

9.1	Lab Report Part 4	15

10	Putting it all together: final system integration!	16

10.1	Lab Report Part 5	16

11	Appendix A: ASCII Table	17

# Objectives

- To learn modular design and development of software.

- To use 9S12C32 hardware timer to generate variable time delays.

- To use hardware-timed delay for debouncing an SPST switch.

- To write a routine for 9S12C32 to interface to and scan an X-Y matrix keypad.

- To learn how to use a lookup table for data conversion.

- To program the 9S12C32’s synchronous communications interface module (SCI) for asynchronous serial data communication.

# Deliverable

Each lab group must submit a written memo-style lab report which refers to six different attachments, where each attachment corresponds to one of the five “Lab Report” sections labeled “Lab Report Part 1” through “Lab Report Part 5”, which are scattered throughout this lab handout.  (See the table of contents.)   The central theme of this lab project is to show that each module was first written (as a subroutine) and separately tested and debugged (by calling it from a very simple test program) before the final step in which all modules are integrated into the final project.

# Reference Materials and Equipment

## References

(1)  TIM16B8 Block User Guide (S12TIM16B8CV1.pdf)

(2)  MC9S12C Family Device User Guide (9S12C128DGV1.pdf)
		(3)  HCS12 Microcontrollers Reference Manual (S12CPUV2.pdf)

(4) Additional MC9S12C32 Reference Manuals  (9S12C32_ZIP.zip)

## Equipment

Freescale CSM12C32 evaluation board (as it was wired onto your breadboard in Lab 1), and a 16-key 4 x 4 row-column matrix keypad (available from the instrument room in C115.)

# Introduction

This project is intended to illustrate how a relatively large project, in this case, the design of a software-debounced keypad entry system with asynchronous serial (RS232) output, can be approached in a top-down, modular, “divide-and-conquer”, fashion.

Along the way, you will gain experience working with the 9S12C32’s hardware timer, software switch debouncing, key depression identification from a row-column keypad, the use of a lookup table, and the programming of the 9S12C32’s asynchronous serial communications interface (SCI) module.

# Modular Programming

Before we get started with the project, let us discuss the concept of modular programming.

## Software modules

A software module is very much like a hardware module, in that it is an easily detachable and separately testable section of code that has just one entry point and one exit point.

In this project, each module will be written as a subroutine. A separate project will be created to test each subroutine using a "“short and simple” test program that calls that subroutine under test and exercises it sufficiently.  Finally, once all of the component subroutine modules have been individually validated, they will be integrated together via a main program that calls each subroutine in turn, which is again yet another project.

## Top-down modular design

The steps in the top-down modular system design (this applies to both hardware and/or software design) are listed below:

- Precisely and fully specify the overall design. When a need arises for a new product, many “gaps” usually exist in the initially (somewhat vague) product description. Many secondary design decisions usually have to be made in order to “fill the gaps”.

- Intuitively break the overall design into smaller subsystems (modules), which are easily designed, debugged, and modified separately from each other. Furthermore, each module should be able to interconnect with the other modules in a clean and straightforward way that requires the fewest possible number of “interconnections”.

- Develop precise and detailed specifications for each module.

- Implement each module exactly as specified. (Perhaps several designers will implement several modules concurrently.) New designers sometimes have trouble following this step. Although they are certainly encouraged to be as clever and creative as possible as they internally design the modules, they MUST NOT creatively modify any of the external module interface specifications in any way. After all, their modules are going to have to integrate with other modules created by others.

- The source (assembly or C) code for each module (usually a subroutine) must be fully described, along with revision number and author, in a “header comment block”. Many companies have a standard header block format that must be followed by any programmers who work for the company. This guarantees that all modules are fully documented right in their source code, permitting these modules to be used easily in future applications as well as in the present one.

- Fully debug and then test each module SEPARATELY until each module fully complies with the specifications that were assigned to it.

- Integrate all modules into a complete system. Then test to see that the original overall system specifications have been fully met. If not, “loop back” to Step 4.

## Documenting a software module: the header block

Each software module that you produce should be accompanied by a software interface definition. This definition typically appears as a header block at the beginning of the module’s source code. The header block consists of a series of comment lines (perhaps set off using rows of asterisks). In general, a well-documented module should contain a header block that includes the following information:

- The program (or subroutine) name.

- Description of its intended purpose and possible application.

- Author’s name, phone number, and company location and job site.

- Date of initial writing.

- For each revision made: revision number, descriptions of revision made and date of revision.

- Text or journal reference(s) to algorithm(s) used in the module.

- Description of all input data needed, and how this input data is to be sent to the module. (There are at least three possibilities: (a) in specified machine registers, (b) in specified RAM locations, and (c) on the stack.)

- Description of output data produced by the module; how each resulting output datum relates to the input data, and where each of the output data can be found after the module is executed.

- Statement of potential problems or side effects of using this software module. Indicate which, if any, internal machine registers are modified (clobbered) by the module.

# Step 1: Write a variable-time-delay subroutine “wait_y_667ns”

In this section you will use one of the 9S12C32’s “Output Compare” registers to write a variable time delay subroutine that meets the software interface specification that appears in the next section.

## Software interface specification for subroutine “wait_y_667ns”

Write a 9S12C32 assembly-language subroutine named “wait_y_667ns” which, when called, will make use of the 16-bit hardware timer (binary up counter) register (TCNT) in conjunction with one of the “Output Compare” timer channel registers (say TC2) to “waste time”, or delay, for Ny*667 nanoseconds before returning control back to the calling program.  No registers may be altered by this subroutine, including the Y register.

The symbol Ny represents an unsigned 16-bit integer (input argument) that is placed in the Y register before the subroutine is called.  This delay routine should use the “software polling” mode of operation; it should NOT be made interrupt-driven (we will work with interrupt-driven output compare applications in Lab Project 4!).

## Using the output compare feature of the 9S12C32’s hardware timing module

Read about the function and programming of Output Compare registers in Section 4.4 of the TIM16B8 Block User Guide (S12TIM16B8CV1.pdf).

Referring to Fig. 4-1 of the TIM User Guide, it should be evident that, in order to use the 16-bit Output Compare register TC2, which is accessible at two adjacent 8-bit addresses, TC2H:TC2L, you must first turn on the timing system by setting the timer enable (TEN) bit (Bit 7) of the TSCR1 (Timer System Control Register 1) to 1.  Setting TEN to one will enable the free-running 16-bit timer (binary up-counter) TCNT, allowing it to begin counting (See Section 3.3.6 of the TIM User Guide).

Since TCNT is 16 bits long, and the 9S12C32 is an “eight bit” microcontroller, meaning that there can only be 8 bits at each memory location, TCNT must be broken up into two adjacent (8-bit) registers (just as with any 16-bit register in the 9S12C32), with the high (most significant) 8 bits of TCNT in register “TCNTHi”, and the lower 8 bits of TCNT in register “TCNTLo”.  We will allow TCNT to “free run”, continuously counting from 0x0000 up to 0xFFFF, and then wrapping back to 0x0000 and repeating the cycle endlessly (until the microcontroller is reset).

The rate at which TCNT is incremented (clocked) is selected by the prescaler bits, PR2:PR0.  These prescale bits may be found in the bottom 3 bits of the Timer System Control Register 2 (TSCR2), as explained in Section 3.3.11 of the TIM User Guide.  Your subroutine should set these prescale bits to %100 (see Table 3-4 in the TIM16B8 Block User Guide) so that the 24 MHz internal clock frequency will be “prescaled” (divided) by 16, and then used to clock the 16-bit up counter, TCNT = TCNTHi:TCNTLo.  Therefore, we may assume that TCNT increments once every 16 / 24 MHz  = 2/3 μs = 667 ns.

You must also set Bit 2 (TIOS2) of the TIOS (Timer Input Capture/Output Compare Select) register  to configure the TC2 (16-bit Timer Channel 2) register (Section 3.3.1 of the TIM User Guide), to function as either an “Output Compare” register as opposed an “Input Capture” register, as explained in Section 4.4 of the TIM User Guide.  .

Since TC2 is configured as an “Output Compare” register, the contents of the TC2 register are continuously compared by hardware with the contents of the free-running TCNT register, and a flag (bit 2 of TFLG1) is set to one when TCNT = TC2, and it stays set until the program writes a “1” to the flag (bit 2 of TFLG1) to reset it to zero.  This flag bit can either be read in a loop, waiting for the flag to be set to one, or it can be configured to interrupt the processor when it becomes set.  Thus the “Output Compare” mechanism permits us to generate precisely timed delays in a program.  We will use this mechanism in its non-interrupt driven mode in this lab project.

If TC2 had instead been configured as an “Input Capture” register (by clearing Bit #2 of the TIOS register), the TC2 register is loaded with the value of TCNT at the precise time that a specified transition occurs (either rising edge, falling edge, or either edge) on the associated Port T input pin, PT2; and at this same instant of time, the associated flag bit (bit 2 of TFLG1) is set.   Thus the “Input Capture” mechanism allows us to determine the precise times between successive edges (transitions) in a digital input signal.  We will work with the Input Capture mechanism in Lab 4.

Next you may want to clear Bit 2 in the Timer Interrupt Enable (TIE) register to ensure that TC2-related interrupts are disabled (See Section 3.3.1 of the TIM User Guide). Because the TIE register is zeroed out of reset, you should not strictly have to perform this step.)

Next you should read the present value of the 16-bit TCNT free running timer, which is accessible at two consecutive addresses, TCNTHi (high byte) and TCNTLo (low byte).  (See Section 3.3.5 of the TIM User Guide.)  Because we know that the TC2 flag in the TFLG1 register will be set as soon as the TCNT register counts up to a value that equals the value loaded in the TC2 register (Section 4.4 of the TIM User Guide), you simply need to add the value in the 16-bit Register Y (which contains Ny, the number of 667 ns time increments we desire to wait) to the current time value found in TCNTHi:TCNTLo.  Then store the result in the TC2 Output Compare Register at TC2Hi:TC2Lo.  This act “schedules” an output compare event to occur at a time equal to Ny*667 ns from the present time, thereby achieving the desired delay.

Immediately after you schedule the new output compare event, you must take action to CLEAR the TC2 Output Compare flag bit by writing a “1” (Not a “0”!) to the position of that flag bit (Bit #2) in the Main Timer Interrupt Flag (TFLG1) register, since TC2 flag has probably already been set from a previous time that the TCNT register “counted past” whatever (garbage value) was previously held by the TC2 register before the desired output compare event was scheduled.  Note from Section 3.3.12 in the TIM User Guide that that writing a zero to a bit position in TFLG1 will NOT change that flag.  Thus the instruction MOVB #4, TFLG1 will clear the TC2 flag, but it will NOT affect the state of any of the other timer interrupt flags (See Section 3.3.12 of the TIM User Guide).  This is important if other output compares are simultaneously being used in a program to perform other timing functions.

Finally, your program must enter a loop that waits until TCNT counts up to a value that equals the contents previously stored in the TC2 Output Compare register.  This event causes the TC2 flag in the TFLG1 register to be set to 1 by the Output Compare hardware at the point in time when TCNTHi:TCNTLo has counted up to the value stored at TC2H:TC2L.  This happens after Ny*667 ns have elapsed.

Note that the Timer Control Registers 1 and 2 (TCTL1 and TCTL2) allow output compare events to cause automatic changes on the corresponding PORT T I/O pin.  (See Section 3.3.8 and Table 3.2 of the TIM User Guide).  However, in this application we do not want to activate this feature, allowing the PORT T pins to be used for simple parallel I/O functions.  Note that TCTL1 and TCTL2 registers are cleared out of reset, and according to Table 3-2 of the TIM User Guide, this will disable the output compare function on the PORT T pins, as we desire.  Therefore your subroutine can either ignore TCTL1 and TCTL2, or you may want to set them to zero up near the beginning of the subroutine, just to be sure that an output compare event will not affect the PORT T pins.

Your subroutine MUST preserve the contents of the A, B, X, Y microcontroller registers if they are altered, since we may later be using this routine to generate a delay in a program that is holding important data in registers A, B, X, or Y at the time our delay subroutine is called.  Therefore, if any of these registers are modified inside subroutine “wait_y_667ns”, their contents must be pushed onto the stack at the start of the subroutine, and then restored (pulled) from the stack, in the reverse order that they were stored, just before returning from this subroutine via the return from subroutine (RTS) instruction.

## Verification of subroutine “wait_y_667ns”

Test your subroutine by calling it using the test program shown in Figure 1.  Note that this program calls wait_y_667ns in order to generate a 50% duty cycle square wave on pin PM0 with a 10 ms period.  Note that code to set the phase-locked loop (PLL) clock generator to a 24 MHz internal bus frequency has been added.  This same initialization is performed by the on-chip serial debugger (which is entered by pressing RESET while S1 is depressed), but it is not performed when a user program is run by itself without the serial debugger (by pressing RESET while S1 is not depressed}.  If the PLL is not initialized using this code, the internal clock defaults to 16/2 = 8 MHz. I have included this initialization code in this program since we want the delay routine and the serial port baud rate to remain the same whether we are running the program under the control of the debugger or without it.  This PLL initialization section may NOT be single stepped through using the serial debugger, since the bus clock changes, and thus so does the serial port baud rate change, while the PLL is temporarily disconnected from the system.  However, we may set a breakpoint at any point beyond this routine and then begin single-stepping from there.

Figure 1. Test program that calls the wait_y_667ns routine.  (You must fill in

the wait_y_667ns routine in the space indicated.)

;******************************************************************************

;    ECE331 Lab 3 Part 1

;    File: sqwave.asm (by KEH)

;    Square Wave Generating Program that calls "wait_y_667ns" subroutine

;    to generate a 100 Hz square wave on pin PM0 that is 5 ms high and 5 ms low.

;******************************************************************************

XDEF sqwave             ; Export 'program entry point' symbol

ABSENTRY sqwave         ; For absolute assembly: mark this as application entry point

INCLUDE 'mc9s12c32.inc'	; Include 9S12C32 register symbol definitions.

ORG $4000

sqwave:     lds  #$3F00             ; Initialize program stack pointer just below UBUG12 stack.

;jmp startpgm

;****Initialize clock generator and PLL for 24 MHz internal bus clock***********

;This initialization is performed by the serial debugger UBUG12, but it is not

;performed when a user program is run by itself (out of RESET without the serial debugger

;In that case, the internal clock defaults to 16/2 = 8 MHz.  That is why I have included this

;code in this program... since we want the delay routine and the serial port baud rate to remain

;the same whether we are running under the debugger or without it!

;

;NOTE: This PLL initialization section may NOT be single stepped through using the serial debugger,

;      since the bus clock changes, and thus so does the serial port baud rate change

;      while the PLL is temporarily disconnected from the system.

;

bclr	CLKSEL,$80        		;disconnect PLL from system

bset	PLLCTL,$40       	  	;turn on PLL

movb	#2,SYNR     	      	;set PLL multiplier

movb	#1,REFDV   	   	;set PLL divider

;PLLCLK = OSCCLK*(SYNR+1)/(REFDV+1)= 16MHz *(2+1)/(1+1) = 24MHz

nop						;NOP delays put here to allow time for

nop						;CRGFLG flag register to become valid.

wt_PLL_Lock:

brclr CRGFLG,8,wt_PLL_Lock	 ;Wait for PLL to lock

bset	CLKSEL,$80      	   		;Connect PLL into system

;*********End of PLL initialization. Now module clk = 24 MHz!

;*******************************************************************************

startpgm:   	movb #1,DDRM           	 ; Make PM0 an output.

ldy  #7500             	 	; Note that 7500*666.667 ns = 5 ms delay time

next_cycle:	bset PTM,1              		; Set PM0 high

jsr wait_y_667ns       		 ; Delay for 5 ms

bclr PTM,1              		; Set PM0 low

jsr wait_y_667ns        	; Delay for 5 ms

bra next_cycle         		 ; go back to create next cycle of 100 Hz sq wave

wait_y_667ns:

…………… put your subroutine here………………

rts

ORG       $FFFE

fdb  sqwave             ; Initialize Reset Vector

A soft copy of the sqwave.asm file shown in Figure 1 is available in the class folder. After you create the template project, copy the sqwave.asm file into this project folder.  Then right click on the template main.asm file and select “Remove” to remove it from the project.  Next right click on the Sources category in the project view and select Add Files.  Select sqwave.asm to add it to the project under the Sources category heading.  You will need to fill in the “wait_y_667ns” subroutine code in the place provided near the end of the file, following the hints of Section 6.2, before this project can be compiled.

## Lab Report Part 1

Run your program and verify its operation using your bench oscilloscope. Be sure to carefully measure the period of the square wave to verify proper timing operation. Include a commented listing of your “wait_y_667ns” delay subroutine along with the test program above.  Obtain the lab instructor’s signature on your assembly listing.

# Investigating the need for software switch debouncing: LED toggling program

Using the same hardware configuration from Lab 2 (Pushbutton SW on PM0 and LED on PM1, write a program that toggles, or changes the state of, the LED every time there is a rising edge on PM0.

## Structure of LED toggling program

Your rising-edge sensitive LED toggling program should follow the following steps

Configure PM0 as input with no internal pullup (pushbutton SW)

Configure PM1 as an output (LED);

next_pass:		Wait here until PM0 goes low (switch closed);

Wait here until PM0 goes high (switch opened);

Toggle PM1 (change state of LED);

Go back to “next_pass”;

If the pushbutton switch did not bounce, each time the switch is pushed (generating a falling edge), nothing should happen to the LED, and each time the switch is released (generating a rising edge), the LED should change state, or toggle.

## Running the LED toggling program with and without contact debounce delay

Now open a new project, and write, compile, and run this program.  When you run this program, you should discover that the LED does not toggle reliably, and furthermore the LED sometimes changes states not just when the pushbutton is released, but also when the pushbutton is pushed!  As was discussed in Lab 1, this undesirable behavior is due to the fact that any mechanical switch contacts bounce for a few milliseconds, both when the button is pushed and also when the button is released.  The LED will appear to change state if an odd number of rising edges are produced after each switch operation as the bounces die out.  On the other hand, if an even number of rising edges are produced, the LED will appear to remain in the same state.

The solution is to call wait_y_667ns both after the step that waits for PM0 to go low and also after the step that waits for PM0 to go high.  (You will want to cut and paste the wait_y667ns subroutine at the bottom of your program from the previous project that generated the 100 Hz square wave.)  Add these two subroutine calls, with Y set to 60000.  Thus the delay subroutine will wait for 60000 * 666.667 ns = 40 ms, which should be quite a bit longer than the worst-case bounce time of a small pushbutton switch.  You should now be able to observe reliable toggling of the LED!

Now try changing Y to lower and lower values of delay.  Find the shortest debounce delay (in milliseconds) for which your particular pushbutton switch appears to work reliably.  This corresponds to the worst-case bounce time of your particular pushbutton switch.

## Lab Report Part 2

Include a hard copy of your commented assembly source code of your (debounced) LED toggling test program.  In your report be sure to indicate the shortest delay time that reliably debounces your particular SPST pushbutton switch.  Demonstrate both unreliable and reliable toggling of the LED to the lab instructor, and get his certifying signature on your assembly source code.

# Debounced keyswitch array input subroutine “getkey”

The keypad we will use for this lab project is a 4x4 X-Y matrix 16-key keypad shown in Fig. 2. It can be checked out from the instrument room.  At first glance, it might seem like the 16 SPST keyswitches would have to be connected to 16 I/O pins on the 9S12C32.  However, because the keypad is arranged in a 4 X 4 “X-Y” matrix (4 rows and 4 columns), we only need to use 8 I/O pins, using a method called “keypad scanning” that is explained below.

The interfacing circuit for the 4 X 4 keypad is shown in Fig. 2.  Note that we have chosen to use the 8 Port T pins to interface to this keypad.  When a keyswitch (represented by a short diagonal line in Figure 2) is depressed, it will close a contact between a column wire and a row wire.

Figure 2.  Interfacing a 4 X 4 keypad to the 9S12C32 microcontroller

To find out when a key is depressed (and also to identify the key’s location in the keypad matrix), the four pins connected to the rows (PT3:0) are initially configured as inputs with internal pull-up resistors enabled, while the four pins connected to the columns (PT7:4) are initially configured as outputs.  It is very important to enable internal (10 kΩ) pull-up resistors on the Port T pins that are configured as inputs.  This is done by setting the “Port T Pull Device Enable” (PERT = $204) register to 0xFF.  (See Section 3.3.1 and Figure 3-5 of the PIM 9C32 Block Guide, S12C32PIMV1.pdf).

The output (column) pins (PT7:4) are then driven low, that is PT7:4 are set to %0000, and the input (row) pins (PT3:0) are repetitively read in a loop.  While no key is yet depressed, all four of the input (row) pins should read high “1”; that is PT3:0 should be read as %1111. But if a key is depressed, the row pin that is connected to the pressed switch is driven low “0”, since the output resistance of a 9S12C32 output pin is much lower than the (internal) 10 k-Ohm pull-up resistor.  Note that the row pin that reads low (all the rest should read high) corresponds to the row that the depressed key is in.

In order to discover the column that the switch is in, the I/O directions of the column pins and row pins should next be reversed.  This time all four of the output (row) pins (PT3:0) are driven low, that is PT3:0 = %0000.  The column pins (PT7:4) may now be read by the M9S12C32 in order to find out which one of these pins is low (certainly one of them should be low, since a key was already found to be depressed). The column pin that reads low (the other 3 inputs should read high) corresponds to the column that the depressed key is in.  The entire procedure for determining the row and column of a depressed key is illustrated in Figure 3. The drawing on the left shows the first step, where the columns (PT7:4) are driven low and the position of the pressed key is read as a “0”on the rows (PT3:0).  The drawing on the right shows the second step, where the rows are driven low (PT3:0) are driven low, and the position of the pressed key is read as a “0” on the columns (PT7:4).

Figure 3.  Two-step scheme for determining the identity of single pressed key (X).

Step 1: Zeros driven onto columns                  Step 2: Zeros driven onto rows

and rows read. Position of 0                and columns read. Position of 0

indicates which row the pressed          indicates which column the pressed

key is in!                                              key is in!

|  | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 0 |  | X |  |  |
| 0 |  |  |  |  |
| 0 |  |  |  |  |

|  | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 0 |  | X |  |  |
| 1 |  |  |  |  |
| 1 |  |  |  |  |

The 4-bit row identification pattern that was read from the bottom half of PORT T when the column lines (PT7:4) were driven to %0000, and the 4-bit column identification pattern that was read from the top half of PORT T when the column lines (PT3:0) were driven to %0000 may be “catenated” together into one 8-bit key identification code, or “keycode”.  (Notice that the upper 4 bits of this code should contain exactly one zero, and likewise, the lower 4 bits of this code should contain exactly one zero.)  A “lookup table” can then be constructed that relates the keycode to the numerical value of the key ($0 - $F).  An incomplete table is provided in Fig. 4 of this handout.  Complete this table on your own. This table must be scanned by your program each time a key is pressed in order to learn the identity (value) of that key.

Figure 4.  Keycodes for each key on the keypad

| PT7 | PT6 | PT5 | PT4 | PT3 | PT2 | PT1 | PT0 | DIGIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 2 |
|  |  |  |  |  |  |  |  | 3 |
|  |  |  |  |  |  |  |  | 4 |
|  |  |  |  |  |  |  |  | 5 |
|  |  |  |  |  |  |  |  | 6 |
|  |  |  |  |  |  |  |  | 7 |
|  |  |  |  |  |  |  |  | 8 |
|  |  |  |  |  |  |  |  | 9 |
|  |  |  |  |  |  |  |  | A |
|  |  |  |  |  |  |  |  | B |
|  |  |  |  |  |  |  |  | C |
|  |  |  |  |  |  |  |  | D |
|  |  |  |  |  |  |  |  | * |
|  |  |  |  |  |  |  |  | # |

You should use the assembly directive “fcb” (form constant byte), or the equivalent “dc.b” (define constant byte) to construct a look-up table in flash ROM memory (not RAM!) that will serve to convert an eight-bit keycode into the key value.  The first part of this look-up table (based on the table of Fig. 4) appears below.

Keycode_Table:  fcb $B7, $7E, $BE ……..

Note that the first entry in the Keycode_Table contains the keycode for the “0” key, the next entry is the keycode for the “1” key, etc.  Thus, when a key is pressed on the keypad and a keycode has been determined by getkey, this table may be searched for a match with the keycode, and the position of the matching element becomes the key value ($0 - $F).

Using the same index offset at which the matching element that was found in the Keycode_Table, index this same distance into a companion ASCII_Table, the beginning of which appears below:

ASCII_Table:   fcb ‘0’, ‘1’, ‘2’,…….

This allows us to find the ASCII code of the key that was pressed.

When a key is found to be pressed, subroutine getkey should wait 40 ms for contact closure bounces to subside, and then wait for the key to be released.  Once all keys are found to be released, the routine should delay for 40-ms before determining the key value from the keycode via a lookup table and returning from the subroutine, since keys can also bounce upon release.

To summarize the above discussion, here is an outline of the “getkey” subroutine:

START Getkey

Loop here until key is pressed;

Delay 40 ms to wait for switch contact bouncing to die out;

Get a 4-bit row position code;

Get a 4-bit column position code;

Catenate the two 4-bit codes together into a “keycode”.

Find which key has been pressed using the Keycode_Table

lookup table

Determine the ASCII code of the key that was pressed

using the ASCII_Table lookup table.

Loop here until key is released.

Delay 40 ms to wait for switch release bouncing to die out.

Return with the ASCII code of the key in Acc B;

END Getkey

Before attempting to debug your program, you should wire the keypad to the 9S12C32, and then check your hardware interface using the MM and MD UBUG2 monitor commands.   The UBUG2 monitor runs on your PC and “talks” to the on-chip serial debug monitor in much the same way that the Metrowerks CodeWarrior debugger does.  It permits memory to be displayed and modified via its MD (Memory Display) and MM (Memory Modify) commands.  The Port T data register (PTT) is at location $240 and its data direction register (DDRT) is at location $242.  The Port T pull enable (PERT) register is located at $204.  You must first download and install UBUG12 on your laptop.  UBUG12 is located in the Lab 1 folder.  Simply click on the UBUG12Installer.msi file to install this simple serial port debug monitor on your laptop.

Here is an example of how you might use UBUG12 to verify your keypad wiring.

mm 204 ff         	;Enable pull-ups on Port T input pins.

mm 242 f0		;Make PT7:4 outputs and PT3:0 inputs.

mm 240 0		;Drive out zeros on PT7:4.

md  240			;Examine value of PTT bits PT3:0.

;With no keys pressed PT3:0 = %1111.

md 240			;If “5” key is pressed, PT3:0 = %1101.

md 240			;Press all other keys and check for proper behavior of PT3:0.

mm 242 0f		;Make PT7:4 inputs and PT3:0 outputs.

mm 240 0		;Drive out zeros on PT3:0.

md  240			;Examine value of PTT bits PT7:4.

;With no keys pressed PT7:4 = %1111.

md 240			;If “5” key is pressed, PT7:4 = %1011.

md 240			;Press all other keys and check for proper behavior of PT7:4.

## Waiting for Port T changes to settle

Due to the high speed at which the 9S12C32 executes instructions, it is necessary to insert a short delay (perhaps about TEN NOP instructions, or you might want to call the wait_y_667ns routine with Y = 25 or so) between when the zeros are driven out on either the high or the low half of Port T and when the other half of Port T is read.  This delay is necessary because it takes time for the voltage on the input pins that are being read stabilize after a change occurs at the output pins of Port T.  This is because there is a significant rise time (perhaps a microsecond) that must be waited out on an input pin due to the RC time constant formed between the internal 10 kΩ pull-up resistance and the parasitic capacitance exhibited by the keypad.  If you forget to put this short “settling delay” into your getkey subroutine, you may find that your getkey subroutine works fine when you are single-stepping through the routine, but that it works unreliably when the program is run at full speed!

## Lab Report Part 3

Demonstrate a short test program that calls subroutine getkey to verify its reliable operation.  Include a commented assembly listing of your program in your lab report.

# Write an asynchronous serial output routine “outchar”

Write a subroutine “outchar” as yet another new project that will send an ASCII code in Accumulator B to the 9S12C32’s serial communication terminal’s display through the DB9 RS232C interface.  This subroutine should be called by a short test program that calls the subroutine to print several ASCII characters over and over again.  This subroutine will use the serial communication interface (SCI) functional block of the 9S12C32 microcontroller to implement RS232 asynchronous serial data communication. (See the “HCS12 Serial Communication Interface Block Guide”, S12SCIV2.pdf.)

The “receive data” input pin PS0 (RxD) is used to receive serial asynchronous data from a terminal..   The “transmit data” output pin PS1 (TxD) is used to transmit serial asynchronous data to a terminal.   The 9S12C32 microcontroller’s two serial communication pins (TxD and RxD) are connected to an RS232 terminal through an on-module interface chip (Max3232CD) that converts the 0 V / 5 V CMOS logic voltage levels to the corresponding +10 V / -10 V RS232 voltage levels.  These RS232-compatible are called “TX” and “RX” on the 9S12C32 module. 	Please study the SCI Block Guide to make sure that you understand why the following registers must be initialized at the beginning of the outchar subroutine in the following way:

					SCIBDL=156	; SCI Baud Rate = 24 MHz / (16 * SCIBDL)

; Note: 9600 Baud = 24 MHz / (16 * 156)

SCICR1=$00	; 1 start bit, 8-bit data, 1 stop bit, no parity

SCICR2=$0C	; transmit and receive enabled, no interrupt

An ASCII character (byte) is sent to the terminal when the data register SCIDRL is written with that byte. But first your subroutine must wait (loop) until the “Transmit Data Register Empty” (TDRE) bit, which is Bit #7 in the SCISR1 register, is a “1”, indicating that the SCI transmit data register is indeed empty, and thus ready to receive new data before the new data byte (in Accumulator B) can be written to the SCIDRL.  The SCI hardware will then automatically begin to shift and transmit the contents of SC0DRL out of the TxD pin to the terminal’s display at the previously configured 9600 bit per second “baud rate”.

Unfortunately, because the UBUG12 serial debug monitor uses the SCI to communicate debugging information between the 9S12C32 module and the host PC, subroutine “outchar” cannot be debugged as can the other subroutines… since both this subroutine and the debug monitor make use of the same SCI block, it would be like asking a doctor to cut out his own appendix!

In order to observe the proper operation of your “outchar” routine, you will have to start a “serial communications terminal” program running on your PC, such as Hyperterminal, which comes with Windows.  You will initially have to set up Hyperterminal by following these steps:

- Make sure that UBUG12 and the Metrowerks Debugger are not running.

- Click on Start – Programs – Accessories – Communications – Hyperterminal

- Enter a connection name such as “ECE331_term”. Hit OK

- In the “Connect To” window, select “Connect Using” COM1 (or COM2, as needed).  Click OK.

- In the properties window, change Bits Per Second: 9600, Data Bits: 8, Parity: None, Stop Bits: 1, Flow Control: None.  Click OK.

- Click on File – Save As, and save your Hyperterminal session to the “Windows Desktop”, so you may click on the resulting desktop icon, rather than having to go through all of these setup steps the next time you want to re-open your Hyperterminal communication window.

- The Hyperterminal communication window should now be visible. Please note that only one program that accesses COM1 can be running at a time.  Thus before you can open the Metrowerks serial debugger, you must close Hyperterminal, and vice versa.

## Lab Report Part 4

Write and demonstrate a short test program that first calls a subroutine called “PLL_init” (See the test program of Fig. 1) that sets the microcontroller’s internal bus clock to 24 MHz.  Then have this test program call the “outchar” subroutine, and have it print several ASCII characters over and over again in an endless loop.  You will want to follow the following steps to observe the proper operation of your program: Power up 9S12C32 module and hold pushbutton switch S1 down while depressing the RESET button.  Make sure the Serial monitor and NOT the debugger option is selected in the CodeWarrior program!  After your program is successfully assembled and linked via the Make button, click on the Green Debug button to load your program into the 9S12C32’s flash program memory.  This also brings up the Metrowerks debugger, but do not do anything else with the debugger. Instead, exit from the debugger (but not from CodeWarrior), and then bring up the Hyperterminal communications window.  Now press the RESET  button to run your program alone without the serial debugger. The desired character stream should be displayed in the Hyperterminal window.

Include a commented assembly listing of your “outchar” subroutine and its associated test program.

# Putting it all together: final system integration!

Now create another new project that uses the short program of Figure 6 to call the various subroutines developed above to implement a simple hexadecimal keypad scanning system, where any sequence of keys (0,1, ....,E,F) can be typed on the keypad, with the corresponding sequence of hexadecimal numbers subsequently displayed on the PC’s Hyperterminal serial communication screen..  The following separately tested software “module” subroutines from the previous portions of this lab should be cut and pasted into the program file after the calling program: PLL_init, wait_y_667ns, getkey, binary_to_ascii, and outchar.

Figure 6.  Final system integration program

Entry:	LDS #$3F00	;Init Stack Ptr near top of RAM just below UBUG Stack

JSR PLL_init	;Init PLL so internal microcontroller bus clock = 24 MHz.

nextchar:	JSR getkey

JSR outchar

LDAB #$0A	;send a line feed to terminal

JSR outchar

LDAB #$0D	;send a carriage return

JSR outchar

BRA nextchar

;…………………………………………………………..

;Subroutines PLL_init, wait_y_667ns, getkey,

;and outchar are all put here

;…………………………………………………………..

## Lab Report Part 5

Your report must include (1) a commented assembly source file of your “wait_y_667ns” delay subroutine along with the test program this final program.  This listing must be signed by the instructor. (2) Include a hard copy of your commented assembly source code of your (debounced) LED toggling test program.  Be sure to indicate the shortest delay time that reliably debounces your particular SPST pushbutton switch. This listing must be signed by the instructor. (3)  Include a hard copy of your commented test program that calls subroutine getkey to verify its reliable operation.  (4) Include a commented assembly source file of your “outchar” subroutine and its associated test program.  (5)  Include a listing of the final program “system integration” program that scans the 4x4 matrix keypad and provides ASCII RS232 serial output.  This file must be signed by the instructor to verify that it was successfully demonstrated.

# Appendix A: ASCII Table

| <TBODY> | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NUL | DLE | Space | 0 | @ | P | ` | p |
| 1 | SOH | DC1 | ! | 1 | A | Q | a | q |
| 2 | STX | DC2 | " | 2 | B | R | b | r |
| 3 | ETX | DC3 | # | 3 | C | S | c | s |
| 4 | EOT | DC4 | $ | 4 | D | T | d | t |
| 5 | ENQ | NAK | % | 5 | E | U | e | u |
| 6 | ACK | SYN | & | 6 | F | V | f | v |
| 7 | BEL | ETB | ' | 7 | G | W | g | w |
| 8 | BS | CAN | ( | 8 | H | X | h | x |
| 9 | HT | EM | ) | 9 | I | Y | i | y |
| A | LF | SUB | * | : | J | Z | j | z |
| B | VT | ESC | + | ; | K | [ | k | } |
| C | FF | FS | , | < | L | \ | l | ¦ |
| D | CR | GS | - | = | M | ] | m | } |
| E | SO | RS | . | > | N | ^ | n | ~ |
| F | SI | US | / | ? | O | _ | o | delete |