ECE554 HW6

- Text Prob. 2.4.  (Show that the second term is small, and that the curve is almost linear, but it is slightly concave upward.)

- Text Prob. 2.5 (Ans: K = 41.7 μV/degree C)

- A thermistor is characterized by the parameters Ro = 10 kΩ, To = 300K, and β = 4000 K.  Find the resistance and the temperature coefficient of this thermistor at T = 270K, 300K, and 330K.   (Ans: Rt = 43.995 kΩ, α =   -5.49%/K;   Rt = 10 kΩ, 
α = -4.4%/K; Rt = 2.976 kΩ, α = -3.673%/K)

- If the capacitance C = Csensor+Ccable+Camplifier in the piezoelectric sensor circuit of Text Fig. 2.10 is quadrupled by adding an external capacitance = 3C in parallel with the piezoelectric transducer, how will the lower corner frequency change? How will the sensitivity of the instrument change?  Explain your reasoning.  (Ans: Corner frequency is only ¼ of what it was, and the sensitivity is only ¼ of what it was.)

- Here are the coefficients taken directly from the NIST document for the Type J (Iron/Constantan) thermocouple, which hold over the temperature range of 0 - 760 degrees Centigrade:

(a) Find the temperature at one junction (Junction 1) of the thermocouple if its other junction (Junction 2) is held at 0 degrees Centigrade using an ice bath, and the voltage read is 4.00 mV?  
(Ans: T = 76.54 degrees C)

(b) If the temperature at the one junction (Junction 1) is 50 degrees C, find the voltage read, assuming that the other junction (Junction 2) is held at 0 degrees Centigrade.
(Ans: x = 2.586 mV)

(c) If Junction 2 of the thermocouple is held at 50 degrees C instead of 0 degrees C, and the other junction (Junction 1) is held at unknown temperature Tx, then find Tx if the thermocouple voltage reads 6.00 mV.   (Ans: Tx = 160.4 degrees C)

(d) Repeat Part (c) if the thermocouple voltage reads -4.00 mV.  (Ans: Tx = -28.41 degrees C)

- A Linear Variable Differential Transformer (LVDT) displacement transducer is 6 cm long, and it is made with three adjacent solenoidal (single-layer close-wound) coils on the same coil form, with the primary coil in the middle, and the two secondary coils on the outside.  Each of the coils has N = 30 turns, a length of 2 cm, and a cross-sectional area of 1 cm2.  Assume that the powdered iron core has a relative permeability μR = 100, thus we may assume that essentially no magnetic flux exists outside of the core in the air, which has μR = 1.  The permeability of free space is μ0 = 4π*10-7 H/m.  The core is 4 cm long, and thus it extends halfway (1 cm) into each of the secondary coils at its 0 – displacement (centered) position.  We require that the primary coil have an impedance magnitude > 400 ohms, so as not to appreciably “load down” the ac source, whose internal resistance Rsource = 50 ohms.

  - Explain why the range of displacements that can be measured by this LVDT is restricted to -1cm < displacement < +1cm.

  - Calculate the self-inductance (Lp) of the primary coil, assuming that the core is never displaced by more than 1 cm from its 0-displacement position.  
(Answer 565.5 μH)

  - Find the minimum permissible frequency of the ac source that drives the primary coil of this LVDT, in order that the stated source loading requirement be satisfied.  (Answer: 112.6 kHz)

  - Find the peak value of the AC output voltage developed at the output terminals of the LVDT if the core is displaced +0.25 cm from its 0-displacement (centered) position, assuming that the source is 10 V peak, that is vsource(t) = 10sin(2πft) volts.  (Answer: Vpeak_output = Vpeak_upper_coil – Vpeak_lower_coil = 2.5 V.)

  - Find the peak value of the AC output voltage developed at the output terminals of the LVDT if the core is displaced -0.5 cm from its 0–displacement position, assuming that vsource(t) = 10sin(2πft) volts.  (Answer: -5 V, where the (-) sign indicates that the output is 180 degrees out of phase with respect to vin(t).)

  - If a source frequency of f = 250 kHz is used, and the output terminals of the LVDT are connected to a diode ring phase-sensitive detector whose output is in turn connected to a first-order RC low-pass filter section with R = 1000 Ω and C = 0.1 μF, find the dc output voltage for the condition of Part d (+0.25 cm displacement).   (Answer: 1.592 V)

  - What is the amplitude of the undesired 500 kHz (fundamental frequency) ripple voltage that gets through the LPF? (Answer: 3.38 mV  Note that the ripple component at the output of the LVDT is negligible, since 1.592 V >> 3.38 mV)