ECE554 HW6

- Text Prob. 2.4.  (Show that the second term is small, and that the curve is almost linear, but it is slightly concave upward.)

- Text Prob. 2.5 (Ans: K = 41.7 μV/degree C)

- Text Prob. 2.6 (Ans: α = -4.4%/K)

- Text Prob. 2.9 (Ans: Cshunt = 0.636 μF, sensitivity is decreased by a factor of about 1300, due to the increase in equivalent capacitance.  Thus we observe that there is a tradeoff between how far the break frequency of the high-pass transducer can be lowered vs. the transducer sensitivity.)

- Text Problem 2.12.  (Note that heatsinking a thermistor will extend its useful linear operating range, and your “thermistor in air” and “heatsinked thermistor” V-I curves should look very similar to those shown for air and water in Fig. 2.13 of the textbook.)

- A Linear Variable Differential Transformer (LVDT) displacement transducer is made with three solenoidal (single-layer close-wound) coils on the same coil form, with the primary coil in the middle, and the two secondary coils on the outside.  Each of the coils has N = 30 turns, a length of 2 cm, and a cross-sectional area of 1 cm2.  Assume that the powdered iron core has a relative permeability μR = 100, thus we may assume that essentially no magnetic flux exists outside of the core in the air, which has μR = 1.  The permeability of free space is μ0 = 4π*10-7 H/m.  The core is 4 cm long, and thus it extends halfway (1 cm) into each of the secondary coils at its 0 – displacement (centered) position.  We require that the primary coil have an impedance magnitude > 400 ohms, so as not to appreciably “load down” the ac source, whose internal resistance Rsource = 50 ohms.

  - Explain why the range of displacements that can be measured by this LVDT is restricted to -1cm < displacement < +1cm.

  - Calculate the self-inductance (Lp) of the primary coil, assuming that the core is never displaced by more than 1 cm from its 0-displacement position.  
(Answer 565.5 μH)

  - Find the minimum permissible frequency of the ac source that drives the primary coil of this LVDT, in order that the stated source loading requirement be satisfied.  (Answer: 112.6 kHz)

  - Find the peak value of the AC output voltage developed at the output terminals of the LVDT if the core is displaced +0.25 cm from its 0-displacement (centered) position, assuming that the source is 10 V peak, that is vsource(t) = 10sin(2πft) volts.  (Answer: Vpeak_output = Vpeak_upper_coil – Vpeak_lower_coil = 2.5 V.)

  - Find the peak value of the AC output voltage developed at the output terminals of the LVDT if the core is displaced -0.5 cm from its 0–displacement position, assuming that vsource(t) = 10sin(2πft) volts.  (Answer: -5 V, where the (-) sign indicates that the output is 180 degrees out of phase with respect to vin(t).)

  - If a source frequency of f = 250 kHz is used, and the output terminals of the LVDT are connected to a diode ring phase-sensitive detector whose output is in turn connected to a first-order RC low-pass filter section with R = 1000 Ω and C = 0.1 μF, find the dc output voltage for the condition of Part d (+0.25 cm displacement).   (Answer: 1.592 V)

  - What is the amplitude of the undesired 500 kHz (fundamental frequency) ripple voltage that gets through the LPF? (Answer: 3.38 mV  Note that the ripple component at the output of the LVDT is negligible, since 1.592 V >> 3.38 mV)