APPENDICES

## APPENDIX A

### Vector algebra

## APPENDIX B

### Coordinate transormations

### APPENDIX C

### Vector calculus definitions

## APPENDIX D

Material properties

## APPENDIX E

### NUMERICAL DETAILS

### Summary - What have we determined so far about the behavior of electromagnetic fields?

#### The electric field is conservative => The work integral of the electric field intensity is zero => LE·dl=0 =>.

#### Because the electric field is irrotational it can be described as the gradient of a scaler potential, E = -ÑV

#### The potential difference between two points is the usual voltage drop of circuit theory, VAB = -BAE·dl

#### The net electric flux emanating from any closed region is equal to the net charge contained within the region, Q=SD·ds; Ñ·D=rV

#### For this reason, the electric field points outward from the charge which is its source.  Electric fields begin and end on charge, but they do not close on themselves.

#### Electric charge is conserved so that I = SJ·ds-dQ/dt,

#### The magnetic field is not conservative => the work integral of the magnetic field intensity is non-zero => LH·dl=IENC. ÑxH = J.

#### For magnetic cores if the path L is restricted to just incrementally short of making one traversal of the core, then no current is enclosed and we can express Ñ·H = 0 and can be expressed as the gradient of a scaler magnetic potential, magnetomotive potential.

#### The magnetic field is perpendicular to the plane formed by the current element and the position vector from the current element to the observation point.  This results in a field which has no beginning or end; the magnetic field is continuous and closes on itself.  As a result, the work integral is in general non-zero.

#### There are no isolated magnetic charges which are the source of magnetic fields.  So the net magnetic flux emanating from a closed region is always zero; SB·ds= 0.

#### Time-varying magnetic fields induce an electric field which is measureable as a voltage across terminals of an inductor.

### Maxwell's Equations - Generalized form of electromagnetic equations

#### Displacement current completes the set of equations.

### Work Integrals

#### Mathematics of closed path line integrals

#### Conservative fields

#### KVL applied to a multi window planar circuit; voltage drops around each loop are found to be zero; moreover, the total voltage drop around the outer (circumscribing) loop is zero.

#### Voltage around a big loop is nothing but a lot of little loops each of which can be considered essentially planar

#### Details of evaluation work integral for a closed path along a differential loop leads to one component of curl.

#### Add two other orthogonal planes to obtain other two components.

#### Definition of curl, relation to conservative fields

#### curlE = 0; first of maxwell's equations

#### Independence of path for a conservative field.

#### Curl meter; irrotational field, circulation of the field does not exist.