;dotpipedfloat.asm  ASM code for dot product with software pipelining

;For floating-point implementation

;cycle 1

MVK	.S1	100,A1	;loop count

||  	ZERO	.L1	A7		;init accum A7

||		ZERO	.L2	B7		;init accum B7

||		LDDW	.D1	*A4++,A3:A2	;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

;cycle 2

||		LDDW	.D1	*A4++,A3:A2	;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

;cycle 3

||		LDDW	.D1	*A4++,A3:A2	;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

;cycle 4

||		LDDW	.D1	*A4++,A3:A2	;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

||	[A1]	SUB	.S1	A1,1,A1	;decrement count

;cycle 5

||		LDDW	.D1	*A4++,A3:A2	;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

;cycle 6

||		LDDW	.D1	*A4++,A3:A2 ;64-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;64-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPYSP	.M1x	A2,B2,A6	;lower 32-bit product into A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product into B6

;cycle 7

||		LDDW	.D1	*A4++,A3:A2	;32-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;32-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPYSP	.M1x	A2,B2,A6	;lower 32-bit product into A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product into B6

;cycle 8

||		LDDW	.D1	*A4++,A3:A2	;32-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;32-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPYSP	.M1x	A2,B2,A6	;lower 32-bit product into A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product into B6

;cycle 9

||		LDDW	.D1	*A4++,A3:A2	;32-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;32-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPYSP	.M1x	A2,B2,A6	;lower 32-bit product into A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product into B6

;cycles 10-109 (loop kernel)

||		LDDW	.D1	*A4++,A3:A2	;32-bit data in A2 and A3

||		LDDW	.D2	*B4++,B3:B2	;32-bit data in B2 and B3

|| [A1]	SUB	.S1	A1,1,A1	;decrement count

|| [A1]	B	.S2	LOOP		;branch to LOOP

||		MPYSP	.M1x	A2,B2,A6	;lower 32-bit product into A6

||		MPYSP	.M2x	B3,A3,B6	;upper 32-bit product into B6

||		ADDSP	.L1	A6,A7,A7	;accum in A7

||		ADDSP	.L2	B6,B7,B7	;accum in B7

;branch occurs here

;cycles 110-124 (epilog)

ADDSP	.L1x	A7,B7,A0	;lower/upper sum of products

ADDSP	.L2x	A7,B7,B0	;

ADDSP	.L1x	A7,B7,A0	;

ADDSP	.L2x	A7,B7,B0	;

NOP				;wait for 1st B0

ADDSP	.L1x	A0,B0,A5	;1st two sum of products

NOP				;wait for 2nd B0

ADDSP	.L2x	A0,B0,B5	;last two sum of products

NOP		3		;3 delay slots for ADDSP

ADDSP	.L1x	A5,B5,A4	;final sum

NOP		3		;3 delay slots for final sum