;dotpnp.asm ASM Code with no-parallel instructions for fixed-point

MVK	.S1	200, A1	;count into A1

ZERO	.L1	A7		;init A7 for accum

LOOP		LDH	.D1	*A4++,A2	;A2=16-bit data pointed by A4			LDH	.D1	*A8++,A3	;A3=16-bit data pointed by A8

NOP		4		;4 delay slots for LDH

MPY	.M1	A2,A3,A6	;product in A6

NOP				;1 delay slot for MPY

ADD	.L1	A6,A7,A7	;accum in A7

SUB	.S1	A1,1,A1	;decrement count

[A1]	B	.S2 	LOOP		;branch to LOOP

NOP		5		;5 delay slots for B