Exam 2

Spring 2008-2009

Name: _____________________________		CM: ______

Section:_____________

Problem 1 (31 pts)	________

Problem 2 (37 pts)	________

Problem 3 (32 pts)	________

Total		________

Be sure to show all work to receive full credit.

Problem 1 – 31 points

- (8 pts) A force, F=300i+180j−240k lb acts at a point (2,0,0).  An axis is described by the unit vector e=0.6i+0.8k and passes through the point (0,3,0).  Distances are in inches.  Calculate the moment of the force about the axis.

Bar Area 0.3 in2

(6 pts) The figure below shows a bar which is 10.0 in long at a temperature of 40oF.  The temperature rises to 100oF.  Calculate (a) the strain in the bar at 100oF and (b) the stress in the bar at 100oF. (Hint: we suggest you check to see whether the gap closes.)

10 in.

E  = 10 x 106psi

α= 20 x 10-6 /oF

Gap = 1.0 in

- (8 pts) Four forces are applied to the corners of a rectangle.  Replace these forces with a single couple.

C=______________________________________

12 N

8 m

90o

30o

5 m

20 N

90o

12 N

30o

20 N

(9 pts) We need to calculate the centroid of the area bounded by the  parabolay=0.5x2, the horizontal line AB, and the y-axis.   We use integration.  Use the differential area shown.

B

A

x

dx

To calculate the centroidal  y-coordinate we need the integral,

y=1AabydA

The differential area, dA,  may be written as

- 0.5 dx

- 0.5 x2 dx

- (0.5 – 0.5 x2) dx

- 0.5 x2 dy

y is equal to

- 0.5x2

- x

- 0.5−0.5x2

- 0.25+0.25x2

The limits of integration are

- a=0, b=1.0

- a =0, b=0.5

- a = x b = 0.5

- a = 0, b = x

Problem 2 – 37 points

A 5mm thick aluminum link is loaded by forces applied to two aluminum pins and one other force as shown.  Relevant dimensions of the link are shown in the figure on the right.  The pin is 6mm in diameter. The material property information for this aluminum is shown in the table below.  Show clear Free Body Diagrams for all forces that are used in stress calculations.

A

B

C

3kN

B

3kN

A

A

1kN

4kN

| 2024-T3 Aluminum |  |
| --- | --- |
| Ultimate Tensile Strength | 480 MPa |
| Yield Strength | 340 MPa |
| Shear Strength | 200 MPa |
| Elastic Modulus | 70 GPa |
| Poisson’s Ratio | 0.33 |
| Density | 2.77 gm/cm3 |

1kN

C

1kN

- Determine the normal stress in the link at section A.

- Determine the Factor of Safety, FOS, with respect to shear failure for the pin.

- The average normal stress at cross section A is_________________ the average normal stress at section B.  (Circle one of the following choices.)

higher than		lower than		the same as

- For a different loading configuration (a set of applied forces that are different than the set shown in the figure) a normal strain of 0.003 was measured at location C.  The normal stress at that location is

- 210 MPa

- 23.3 MPa

- 42.8 MPa

- It is not possible to determine the answer from the given information

Problem 3 – 32 points

y

x

50mm

30mm

100mm

50mm

25mm

Locate the x and y centroids of the shaded region.

Hint: A table of the centroids of simple shapes is on the back of your equation sheet.