Name: 			  CM Box: ___________

Circle your section:

Sanders – 05		Sanders – 06		Lui – 07		Lui – 08

ES 202

Fluid & Thermal Systems

Examination I

December 17, 2004

| Problem | Score |
| --- | --- |
| 1 | /70 |
| 2 | /30 |
| Total | /100 |

Show all work for credit

Open table ONLY

Laptops allowed

Problem 1  (70 points)

- Determine the thermodynamic properties for Refrigerant 134a in the unshaded boxes of the table below, up to 4 significant figures.  You may use the next page as work space.  Use the following abbreviations when specifying any verbal description:

CL	=  compressed (subcooled) liquid

SL	=  saturated liquid

SM	=  saturated mixture

SV	=  saturated vapor

SHV	=  superheated vapor

NA	=  not applicable

INSUF	=  insufficient information

| State | Phase | Pressure, P (kPa) | Temperature, T (oC) | Specific Volume, v (m3/kg) | Specific Enthalpy, h (kJ/kg) | Quality, x |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 100 | 40 |  |  |  |
| 2 | SL | 100 |  |  |  |  |
| 3 |  | 100 | -26.43 |  |  |  |
| 4 |  | 100 |  | 0.10 |  |  |
| 5 |  | 1000 | 30 |  |  |  |
| 6 |  |  | 30 |  | 277.17 |  |
| 7 |  |  | 30 |  |  | 0.8 |
| 8 | SV |  | 30 |  |  |  |

- Locate all identifiable states (1 – 8) on the P-v and T-v diagrams below.

T

P

v

v

30 oC

100 kPa

Problem 2  (30 points)

Given air at two different states:

State 1:  T1  =  500 K, P1  =  100 kPa  (ideal gas)

State 2:  T2  =  1500 K, P2  =  1 MPa

- Is the ideal gas model a reasonable approximation to State 2?  Show your reasoning.

- Determine the following changes in thermodynamic properties using the most accurate method available to you:

  - specific enthalpy:     h2 – h1  =   	 kJ/kg

  - specific entropy:     s2 – s1  =   	 kJ/kg-K

  - specific volume:     v2 – v1  =   	 m3/kg