Name: 			  CM Box: ___________

Circle your section:

Lui – 01		Lui – 02		Richards – 03		Richards – 04

Sanders – 05		Sanders – 06		Mech – 07

ES 202

Fluid & Thermal Systems

Examination I

December 19, 2005

| Problem | Score |
| --- | --- |
| 1 | / 40 |
| 2 | / 33 |
| 3 | / 27 |
| Total | /100 |

Clearly show all work for credit.

Open table ONLY

One side of an 8.5” x 11” equation sheet is allowed.

Laptops allowed

Problem 1  (40 points)

- (31 points)  Complete the table of thermodynamic property information shown below for Water (H2O).  Skip the shaded boxes. You are encouraged to use the blank space on this page and the next page as work space.

Numerical answers should be provided with an accuracy of at least 4-significant figures. (Round-off can get you in trouble!)

Please use the following abbreviations as required:

| CL = compressed (subcooled) liquid SL = saturated liquid SM = saturated two-phase mixture SV = saturated vapor SHV = superheated vapor | NA = not applicable INSUF = insufficient information |
| --- | --- |

| State | Pressure P (MPa) | Temperature T (oC) | Quality, x | Specific Volume υ (m3/kg) | Specific Enthalpy h (kJ/kg) | Phase |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4.0 |  | 0.3 |  |  |  |
| 2 | 4.0 | 200 |  |  |  |  |
| 3 | 4.0 |  |  | 0.0715 |  |  |
| 4 | 4.0 | 300 |  |  |  |  |
| 5 |  | 300 |  |  |  | SV |
| 6 |  | 300 |  |  | 2543 |  |

Workspace

Problem 1 (continued)

- (9 points)  On the following P-v diagram,

  - Carefully sketch the 4.0 MPa isobar and the 300 oC isotherm.

P

υ

4.0 MPa

Carefully locate all six states (1-6) from Problem 1 (a).  Take care to correctly locate the states relative to the saturation curves and the appropriate isotherm and isobar.

Problem 2  (33 points)

Analyze the AIR compressor in the diagram at the right.  The AIR enters the compressor at 100 kPa and 290 K.  It exits at 550 K and 683 kPa.  Treat the AIR as an ideal gas and apply standard assumptions for a compressor.  Use the ideal gas table for air (Table A-21).

ⓔ

.

(a)	The power required to drive the compressor is 375 kW.  What is the mass flow rate of the air at the inlet, in kg/s?

Win

(b)	What is the entropy generation rate, in kW/K?

ⓘ

(c)	Prove that the AIR can be modeled as an ideal gas at the exit state.

Problem 3  (27 points)

A piston-cylinder device contains air.  Initially the air occupies 0.34 m3 at 575 kPa and 167 C.  The air undergoes a constant-pressure heat transfer process until it reaches a temperature of 437 C.  Assume air is an ideal gas.  Find the direction and magnitude (in kJ) of the heat transfer for this process.