Linkages and Relative Acceleration

(by Z. Chambers)

Recalling our earlier discussion of relative velocity for objects undergoing general plane motion, we developed three techniques for relative the velocity of any two points.

1.	Instantaneous Center : if you know the direction of the velocity of two points on the body, you can locate the IC and use a scalar analysis to obtain the velocity via

vA=ωABrA/IC ; vB=ωABrB/IC etc.

2.	Vector Algebra : define all velocities in terms of i,j,k components and then equate the components to obtain two equations

vB=vA+vB/AvA+ωAB×rB/A

3.	Vector Diagram : draw a vector triangle and measure the results (we don’t do this anymore!)

The rate form equations we will be applying to solve linkage problems will introduce the angular acceleration of the body along with the accelerations at common points, thus we need to develop a technique to relate accelerations for points on rigid bodies undergoing general plane motion.  We shall begin by time-differentiating the vector algebra equation for relative velocity:

ddtvB=ddtvA+vB/A

where

aA corresponds to the translational acceleration of A

aB/A corresponds to the rotational (relative) acceleration about A

Note:

aB/A=ddtvB/A=ddtωAB×rB/AαAB×rB/A+ωAB×vB/AαAB×rB/A+ωAB×ωAB×rB/A

Thus the relative acceleration aB/A has two components:

αAB×rB/A		is the tangential component of the relative acceleration

ωAB×ωAB×rB/A	is the normal component of the relative acceleration

Thus, for general plane motion

aB/At=αAB×rB/A	mag = αABrB/A;	direction is  to rB/A

aB/An=ωAB×ωAB×rB/A	mag = ωAB2rB/A;	direction is towards A

Notes on Solving Problems:

Plane Motion: 	in 2d, ω=ωk so ωAB×ωAB×rB/A=−ω2rB/A

Pinned Joints: 	the pin has the same absolute acceleration regardless of the body you isolate

Gears :	tangential components of acceleration are equal, normal components are different