Review Problems - Final Exam

1.	The 8 kg rod AB is attached by pins to two 5 kg uniform disks as shown.  The assembly rolls without sliding on a horizontal surface.  If the assembly is released from rest when =60 deg, determine

a)	the angular velocity of the disks when =180 deg

b)	the force exerted by the surface on each disk at that instant

(taken from Dynamics by Beer and Johnston, Fifth Edition)

ans:  ω=8.44 rad/s CCW, N=122.5 N↑

2.	In a study of head injury against the dashboard of a car during sudden or crash stops where lap belts without shoulder straps are used, the segmented human model shown in the figure is analyzed.  The hip joint O is assumed to remain fixed relative to the car and the torso above the hip is treated as a rigid body of mass m pinned at O.  The center of mass of the torso is at G with the initial position of OG taken as vertical.  The radius of gyration about O is kO.  If the car is brought to a stop with a constant deceleration a, determine the velocity v relative to the car with which the model’s head strikes the dashboard.

Substitute the values:

m=50 kg r=450 mm r=800 mm kO=550 mm θ=45∘ a=10g

(taken from Dynamics by Meriam and Kraige, Fourth Edition)

Hint: IO=IG+mr2

ans:  v=11.73 m/s

3.	An intermittent-drive mechanism for a perforated tape F consists of the link DAB driven by the crank OB.  The trace of the motion of the finger at D is shown by the dashed line.  Determine the magnitude of the acceleration of D at the instant shown when both OB and CA are horizontal.  Crank OB has a constant clockwise angular velocity of 120 rpm.

(taken from Dynamics by Meriam and Kraige, Fourth Edition)

ans:  aD=1997 mm/s2

4.	The uniform concrete block, which weighs 171 lb and falls from rest in the horizontal position shown, strikes the fixed corner A and pivots around it with no rebound.  Calculate the angular velocity of the block immediately after it hits the corner and the percentage loss n of energy due to the impact.

(taken from Dynamics by Meriam and Kraige, Fourth Edition)

ans:  ω=1.593 rad/s CCW, n=91.7 %