Review Problems - Final Exam

1.	The 8 kg rod AB is attached by pins to two 5 kg uniform disks as shown.  The assembly rolls without sliding on a horizontal surface.  If the assembly is released from rest when =60 deg, determine

a)	the angular velocity of the disks when =180 deg

b)	the force exerted by the surface on each disk at that instant

(taken from Dynamics by Beer and Johnston, Fifth Edition)

ans:  ω=8.44 rad/s CCW, N=122.5 N↑