Exam 1

Spring 2010-2011

Name: _____________________________		CM: ______

Section:_______________________

Problem 1 (35 pts)	________

Problem 2 (30 pts)	________

Problem 3 (35 pts)	________

Total		________

Be sure to show all work to receive full credit.  However, “given” and “find” are not necessary.

Problem 1 – Short Answer -- 35 points

z

(a) Point Q has coordinates (2,2,2).  A force, A, whose magnitude is 70 N, acts at Q along a line toward Point P (5, -4, 0).

Q  (2,2,2)

A

P  (5,-4,0)

y

x

- Write force  A in Cartesian vector form.

- What angle does force A make with the x-axis?  Give your answer in degrees.

(c)  An aluminum bar has the following specifications:

| Length | 10 in |
| --- | --- |
| Diameter | P P 0.505 in |
| Cross sectional area | 0.2 in2 |
| Modulus of Elasticity | 1.0 x 107 lb/in2 |
| Yield Strength | 38,000 lb/in2 |
| Ultimate Tensile Strength | 65,000 lb/in2 |
| Coefficient of Thermal Expansion | 12.5 x 10-6/oF |

- If the bar carries an axial force of 3100 lb, calculate the axial strain.

- Calculate the smallest force which would leave a noticeable plastic (permanent) deformation after unloading.

- What is the smallest applied force which would cause the bar to break?

- If we heat the bar up by 50oF, what is the change in length of the bar?

P

P

(d)  A 1/8” diameter wooden dowel is sliced at a 60o angle, and then it is joined with glue.  What is the normal stress on the glued plane due to an applied force of 100 pounds?

60o

Problem 2 – 30 points

A window washer uses a system of frictionless and massless pulleys to hoist himself up the side of a building. The ropes in the pulley system are light and inextensible. The window washer has a weight of W and the platform on which he stands has negligible weight.

- Find the force with which the window washer pulls on the rope in terms of the weight W.

- Find the tension in Rope 2 in terms of the weight W.

- Find the force the window washer’s feet exert on the platform in terms of the weight W.

Rope 2

Problem 3 – 35  points

Three cylindrical bars (I, II, and III) are attached to each other and to a wall at one end, and are loaded as shown below.  Sections I and III are hollow with an inner diameter of 20 mm and an outer diameter of 30 mm.  Section II has a solid, circular cross-section of an unknown diameter.

- Find the reaction force at the wall.

- Determine the average normal stress in section III.

I

III

II

I

If the stress in section II cannot exceed 50 MPa, determine the minimum diameter of the solid cylinder in section II.

50 kN

100 kN

30 kN

I