Project 2,3,4 Possibilities for 2008-09

Mini Air Compressor

Project 2 Dissection and Diagnosis

Part1 (in class) - Dissection and Identification

- Disassemble the air compressor.

- Make partial parts list and identify some manufacturing features (ejection pin parks in a metal and in a polymer part, parting lines, single pull vs slides, etc.)

Part 2 (outside of class) – Manufacturing method Diagnosis

- For select parts (crank counter weight, piston, housings, crankshaft, cylinder,…), deduce the likely method of fabrication using a differential diagnosis technique.  Provide written and pictorial evidence.

Project 3 Redesign for a Different Process

For one of the parts in the compressor, redesign as a (stamping, extrusion, etc)

- Make a correctly dimensions and tolerance detail drawing ready for submission to the supplier.

Project 4 Tolerance Stack-Up

Provide drawings of Piston, wrist pin, connecting rod, crank (counterweight, pin, shaft), bearing, housing, cylinder.

Modify dimensions and tolerances such that

- parts can assemble (press fit interference is < __)

- moving parts can move (minimum clearance >__)

- Piston does not go past the ends of the cylinder at top dead center or bottom dead center