Lesson 1 Diagnostic Quiz

- Find the lift coefficient of a Learjet travelling at 400 ft/sec at 20,000 ft (density = 0.001267 slug/ft3).  Assume a wing planform area of 265 ft2, a weight of 23,000 lbf, and steady, level, unaccelerated flight (SLUF).

- Find the thrust required for this Learjet if the drag coefficient is 0.14.

- What type of drag is probably dominant on this airplane in this condition?  Why?

- Find the stall speed at sea level (density = 0.002377 slug/ft3) if CLmax = 1.7

- What are the two sources of all aerodynamic force?

6.  Write the Bernoulli equation and use it to find the pressure difference between the freestream and a point on the top of the wing of the Learjet in problem 1 if the velocity at that point on top of the wing is 500 ft/sec.