ME 305 Wind Tunnel Exercise

Objectives

- Clarify when Patm ≈ Pstagnation (open-circuit wind tunnel) and when Patm ≈ Pstatic (actual flight)

- Compute airspeeds for a wind tunnel flow and a free jet flow (resembling actual flight in the atmosphere).

- Determine the error in assuming incompressible flow for low-speed flows.

HW (due Friday 25 Mar 05, submit this sheet and calculations on separate sheets):

Find true airspeeds in wind tunnel and from hair dryer assuming incompressible flow

Find true airspeeds in wind tunnel and from hair dryer assuming compressible flow

Measurements:

Wind Tunnel (for each pressure measurement, be sure to indicate whether it is positive or negative)

| Approx. Indicated Airspeed (mph) | Patm - Pstatic (in H2O) | Patm - Pstagnation (in H2O) | Vtrue (ft/sec) (incompress.) | Vtrue (ft/sec) (compress.) |
| --- | --- | --- | --- | --- |
| 60 ± 10 |  |  |  |  |
| 120 ± 10 |  |  |  |  |

Hair Dryer (for each pressure measurement, be sure to indicate whether it is positive or negative)

|  | Patm - Pstatic (in H2O) | Patm - Pstagnation (in H2O) | Temp (°F) | Vtrue (ft/sec) (incompress.) | Vtrue (ft/sec) (compress.) |
| --- | --- | --- | --- | --- | --- |
| Low |  |  |  |  |  |
| High |  |  |  |  |  |

Room Barometer

| Patm (in Hg) | Tatm (°F) |
| --- | --- |