Let us consider a circuit with two energy storage elements.  In this problem, we concentrate on finding the initial conditions of the circuit.

For the circuit shown below, the switch has been opened for a long time and it is closes at t=0. Determine the initial values of the inductor and capacitor voltages and currents. iL(0+),vL(0+), ic(0+),vc(0+).

An instantaneously change in inductor current is impossible because the infinite voltage would be required. This means the inductor current must be continuous. vL=LdiLdt

iL(0+)=iL(0−)vc(0+)=vc(0−).

Electric charge in a capacitor can not change instantaneously. So q(t) must be continuous over time. Recall that q(t)=Cv(t). Thus the voltage across a capacitor cannot change instantaneously.

Let’s take a look on the circuit immediately before the switching. The circuit is in steady state mode since the source is dc and the switch has been open for a long time. So the capacitor behaves as open circuit and the inductor behaves as a short circuit.

iL(0+)=iL(0−)=0vc(0+)=vc(0−)=−40V

KVL loop 1: voltage drop +

−80+10i(0+)+(−40)=0i(0+)=12mA

ic(0+)=i(0+)=12mA

KVL loop 1: voltage drop:

−(−40)+20×0+vL(0+)=0

vL(0+)=0V