Example 3.

Consider the RL circuit, the switch closes at t=0. Assume the initial current through the inductor isI0. Our goal is to find the inductor current iL(t), as t>0

We can apply the Kirchoff’s voltage law to solve the problem.

−V+RiL+LdiLdt=0

The circuit is described by the first order differential equation. The equations can be written as:

We first separate the variable i and t.diLdt=VL−RLiL multiply both sides by –R/L

diLVL−RLiL=dtdiL−LRVL+iL=−RLdtdiL−VR+iL=−RLdt

Taking the integration and we get

Consequently we get,

lni(t)−VRi(0)−VR=−RLt

iL(t)=VR+(I0−VR)e−RLt

iL(t)=If+(I0−If)e−tτ, τ=LR

Key steps to calculate the step response of a first order circuit:

- Find the initial current I0 at t=0+and

the final current If as t→∞

- Find the time constant  τ=LR

- iL(t)=If+(I0−If)e−tτ