Homework 3

ECE 207 – Electrical Engineering

Due: 9/20/2012

- A 60 Hz ideal transformer is rated 220/110 V. An inductive load of Z2 = 10 + j10 Ω is

connected across the low voltage side. Calculate:

  - Primary and secondary current phasors in polar form

  - Load impedance referred (reflected) to the primary side

  - Power supplied by a 220 V source

- If ZL = 80 /60o Ω, then find the input impedance Zab in the following circuit.

- A 25 kVA, 440/220 V, 60 Hz transformer has the following parameters:

R1 = 0.16 Ω		X1 = 0.32 Ω		Rc1 = 270 Ω		Xm1 = 100 Ω

R2 = 0.04 Ω		X2 = 0.08 Ω

The transformer delivers 20 kW at 0.8 power factor lagging to a load on the low voltage side.

- Draw the transformer exact equivalent circuit with the parameter values specified on it

- Draw the reflected and combined circuit for this transformer

- Calculate the high voltage side voltage (Vg)

- A 50 kVA, 2400/240 V, single-phase transformer was tested and the following results were

obtained:

Test Type			Voltage (V)		Current (A)		Power (W)

Open Circuit		      2400		      0.54		     186

Short Circuit		        48		    20.83		     620

- Find the transformer model resistance and reactances

- Draw the transformer equivalent circuit with parameter values specified on it

- For transformer of Problem 4, a load of 50 kVA at 0.8 power factor lagging is connected to

the secondary (low voltage) side. Determine the voltage regulation and efficiency.