Homework 4

ECE 207 – Electrical Engineering

Due: 9/27/2012

- A three-phase, 10 hp, 208 V, 50 Hz, 4 pole, Y-connected induction motor delivers rated

output power at a slip of 2%. Determine:

  - Synchronous speed

  - Rotor speed

  - Synchronous speed and rotor speed, if the motor had 2 poles

- A three-phase, 25 hp, 230 V, 60 Hz induction motor draws 60 A from the source at 0.866

lagging power factor. The motor losses are:

- Stator copper loss, Pcu1 = 850 W

- Magnetic core loss, Pcore = 450 W

- Rotor copper loss, Pcu2 = 1050 W

- Rotational loss, Prot = 500 W

Find:

  - Air gap power, Pag

  - Slip, s

  - Mechanical power developed, Pdev

  - Output power, Pout

  - Efficiency of the motor, η

- A three-phase, 50 hp, 460 V, 60 Hz, 4 pole, induction motor has the following data:

R1 = 0.1 Ω			X1 = 0.35 Ω

R2 = 0.125 Ω			X2 = 0.40 Ω

Rc = 1200 Ω			Xm = 100 Ω

This motor operates at a slip of 2.5%. Ignoring windage losses, calculate:

  - Line current, I1

  - Input power, Pin

  - Input power factor, pf

  - Efficiency, η

  - Starting current, Istart

  - Starting power factor, pfstart

- A three-phase, 5 hp, 208 V, 4 pole, 60 Hz induction motor is subjected to no-load, blocked-

rotor, and DC tests. The following data was collected from these tests:

Variable	No-Load Test		Blocked-Rotor Test		DC Test

Voltage (V)	       208			35			    20

Current (A)	           4			12			    25

Power (W)	       250		           450

Find:

- Parameters of the equivalent circuit

- Draw the equivalent circuit with parameter values specified on it

- For the motor of Problem 4, it is desirable to improve the power factor to 0.98 lagging.

If the slip is 5% and terminal voltage is 208 V, then calculate:

  - Reactive power of the capacitor/phase for power factor correction

  - Capacitance of the Y-connected capacitor/phase for power factor correction