Homework 5

ECE 207 – Electrical Engineering

Due: 10/4/2012

- Two conductors were run in parallel in a duct with a distance of 3 inches between them.

Conductor #1 is carrying a current of 50 A, and conductor #2 is carrying a current of 300 A

in the opposite direction. If the conductors are attached to the opposite sides of the 50 ft

duct, then calculate:

  - Force exerted on the duct for the above load currents

  - Force exerted on the duct, if the short circuit current of conductor #1 is 750 A

  - Force exerted on the duct, if the short circuit current of conductor #2 is 3,000 A

  - Maximum feasible force on the duct

- The shown magnetic circuit has the following dimensions:

  - Cross-sectional area of 16 cm2

  - Average core length of 40 cm

  - Number of turns of 350

  - Core relative permeability of 50,000

  - Magnetic flux density of 1.5 Tesla

Determine:

- Magnetic flux φ

- Total flux linkage λ = N φ

- Required current through the coil

- Inductance of the coil

- An air gap of 0.05 cm in length is cut in the right leg of the magnetic circuit of Problem 2

that changes the magnetic flux density. If a current of 1.2 A flows through the coil, then

calculate:

  - Magnetic flux φ

  - Total flux linkage λ = N φ

  - Magnetic flux density

  - Inductance of the coil

- Calculate the force in the air gap for Problem 3, if:

  - Current is 1.2 A

  - Current is 2.4 A

  - Current is 3.6 A

What can you conclude from this exercise?

- A cylindrical actuator is to be designed with the following parameters:

  - Diameter is 5 cm

  - Air gap length is 0.4 cm

  - Attached weight is 0.5 kg

Calculate the number of turns, if:

a.   The coil current is 1.5 A

b.   The coil current is 3 A

What can you conclude from this exercise?