ECE 370

HW SET #2

- A single-phase 2400/120 V transformer is rated at 6 kVA, and has an equivalent circuit impedance referred to the secondary side of Ze2 = 0.0125 + j0.25 Ω. This transformer is connected to a source that produces a 110 /0o V at the load with the full load current at 0.9 lagging power factor. Calculate:

  - Full load current phasor at the load side in polar form

  - Load impedance in polar form

  - Full load current phasor at the source side in polar form

  - Transformer equivalent impedance referred to the primary side in rectangular form

  - Source voltage phasor in polar form

- The transformer of Problem 1 now is connected to source voltage of 2500 /0o V. The load impedance is still ZL = 2.2 /25.84o Ω. Calculate:

  - Load impedance referred to the primary side in polar form

  - Source current phasor in polar form

  - Load current phasor in polar form

  - Load voltage phasor in polar form