ECE 472 - POWER SYSTEMS II

Homework #2

The operating equation for a hinged armature relay with a shading ring is:

Fnet=Kφ(NIOP)2la2 −KS

= FOP - FRES

The operating current supplied to the relay is purely sinusoidal. Assume K  and KS are constant over the range of all possible operating conditions.

1.	Solve for the relay current IOP in terms of K, N, la, and KS when Fnet= 0.

2. 	Initially, with the relay in reset position (“b” contacts closed), the following values are given:

a. Number of turns in the operating coil: N=50 turns

b. Length of the air gap: 1a = 0.01 m

c. KSKφ= 15,000   A−turnsm

Calculate the relay current IPU at which Fnet  = 0.

3.	As the relay goes from the reset position to the picked-up position the length of air gap decreases.  Assume IOP is constant during this time.  What happens to the net operating force Fnet as this occurs?    Why?

4.	a.  Assume 1a = 0.005 m when the relay “a” contacts are closed.  Calculate

the value of relay current at which the relay drops out (dropout current =

IDO).

b. What is the dropout to pickup ratio for this relay?

5.	An adjustment is made to the restraining spring such that the value of Ks increases by a factor of 4.   Recalculate the relay current IPU at which Fnet = 0 with the relay in reset position.  (Use 1a = 0.01 m)

6.  	The restraining spring (in part 5) is reset to its original position. An adjustment is made such that 1a = 0.04 m in reset position.  Recalculate the relay current IPU at which Fnet = 0.

7.	If number of turns in the relay operating coil is increased, what will be the effect on pick-up current?   Why?